TizenRT Build Failure Analysis

bk7239n/hello CI failure after merge of PR #7312

| Field | Value |
| --- | --- |
| Report ID | adbtrr_work_5_issue |
| Date | 05 August 2026 |
| Repository | Samsung/TizenRT |
| Broken by | PR #7312 — merged commit 5dad7bae2b12450354791bfe373a43fa4b704f96 |
| PR title | apps/examples/board_specific_demo/secure_storage_demo: Add support of secure storage test for TP1X plus board |
| Merged on | 04 August 2026 |
| CI system | CircleCI — job bk7239n_hello |
| Failing stage | pass2 link (arm-none-eabi-ld) |
| Severity | Blocker — master build is broken for all bk7239n configurations |
| Status | Root cause identified, fix implemented and build-verified |

# 1. Executive Summary

After PR #7312 was merged into Samsung/TizenRT master, the CircleCI job bk7239n_hello fails. The failure is not a compile error — every translation unit compiles cleanly. The build dies at the final kernel link step with an undefined reference.

PR #7312 added a new source file os/se/armino/bk_security_api_wrapper_tz.c and registered it unconditionally in os/se/armino/Make.defs. One function in that file, bk_verify_flash_protect(), calls bk_flash_get_protect_type() — a Beken flash API that is declared in the public board header but has no implementation anywhere in the in-tree Beken SDK. The reference therefore survives compilation and only surfaces when tinyara.axf is linked.

The fix is to read the flash status register directly through bk_flash_read_status_reg(), which is implemented in the flash driver, and to test the block protection bits — exactly the approach the existing Ameba wrapper already uses. A full build of bk7239n/hello with this change links successfully.

# 2. Failure Signature

Reproduced locally at master HEAD 5dad7bae by running the same commands as the CI job:

$ cd os/tools && ./configure.sh bk7239n/hello

$ cd os && make

The build proceeds normally until the kernel link:

LD: tinyara

arm-none-eabi-ld: build/output/libraries/libse.a(bk_security_api_wrapper_tz.o):

in function `bk_verify_flash_protect':

os/se/armino/bk_security_api_wrapper_tz.c:128:

undefined reference to `bk_flash_get_protect_type'

make[1]: *** [Makefile:208: ../build/output/bin/tinyara.axf] Error 1

make: *** [Makefile.unix:483: pass2] Error 2

# 3. Root Cause Analysis

## 3.1 What PR #7312 introduced

The PR changed twelve files. Three of them are relevant to this failure:

| File | Change |
| --- | --- |
| os/se/armino/bk_security_api_wrapper_tz.c | New file. Provides ns_flash_erase(), bk_ss_flash_read() and bk_verify_flash_protect() for the secure storage demo. |
| os/se/armino/Make.defs | Added the new file to SE_CSRCS, inside the existing ifeq ($(CONFIG_SE_ARMINO),y) block. |
| build/configs/bk7239n/hello/defconfig | Enabled CONFIG_EXAMPLES_SSTORAGE, CONFIG_SECURITY_API and CONFIG_SECURITY_SS. |

## 3.2 The unresolved symbol

The offending function, as merged:

/* os/se/armino/bk_security_api_wrapper_tz.c:124 */

int bk_verify_flash_protect(void)

{

flash_protect_type_t protect_type;

protect_type = bk_flash_get_protect_type();      /* <-- never defined */

printf("Flash Protect Type: %d\n", protect_type);

if (protect_type != FLASH_PROTECT_NONE) {

return 1;

} else {

return 0;

}

}

bk_flash_get_protect_type() is declared at os/board/bk7239n/src/include/driver/flash.h:131, so the call compiles without so much as a warning. There is, however, no definition of it in the tree.

In os/board/bk7239n/src/middleware/driver/flash/flash_driver.c the protection logic exists only as file-static helpers and a static state variable:

static flash_protect_type_t s_flash_runtime_protect_type = FLASH_PROTECT_NONE;   /* line 190 */

static uint32_t flash_get_protect_cfg(flash_protect_type_t type)                 /* line 563 */

static void     flash_set_protect_cfg(uint32_t *reg, uint32_t cfg)               /* line 579 */

static void     flash_set_protect_type(flash_protect_type_t type)                /* line 620 */

Neither bk_flash_get_protect_type() nor bk_flash_set_protect_type() has a public definition. Both are header-only declarations. Every other Beken API the new file uses (bk_flash_erase_sector, bk_flash_read_bytes, bk_flash_write_bytes, bk_flash_read_status_reg, bk_flash_write_status_reg, bk_flash_write_enable, bk_flash_write_disable) is properly implemented, which is why this is the only unresolved symbol.

## 3.3 Why the other helpers in the file do not fail

The new file also defines ns_flash_read(), ns_flash_write(), ns_setstatusbits() and ns_printf(). All four are static and unreferenced, so the compiler discards them before they can pull in any symbol. The bk7239n warning set includes -Wno-unused-function, so they do not even produce a diagnostic. Only bk_verify_flash_protect() is externally visible and reachable, so only its unresolved call reaches the linker.

## 3.4 Scope — all four bk7239n configurations are affected

The new source file is gated on CONFIG_SE_ARMINO, not on CONFIG_EXAMPLES_SSTORAGE. All four bk7239n defconfigs set CONFIG_SE_ARMINO=y, and all four have a CircleCI job, so all four break identically. bk7239n/hello is simply the one that was reported.

| CI job | CONFIG_SE_ARMINO | Expected result |
| --- | --- | --- |
| bk7239n_hello | y | Link failure |
| bk7239n_loadable_apps | y | Link failure |
| bk7239n_xip_all | y | Link failure |
| bk7239n_loadable_all | y | Link failure |

## 3.5 Why the PR appeared to have green checks

The commit is authored 09 February 2026 but committed 04 August 2026 — the PR was open for roughly six months. Master history is linear: the parent of 5dad7bae is 474bf67c, the previous master tip. The change was therefore rebased onto current master at merge time, and the passing checks recorded on the PR belong to an earlier SHA built against an older base, not to the commit that actually landed.

# 4. Solution

## 4.1 Approach

Stop calling the non-existent API. Read the flash status register with bk_flash_read_status_reg() — which is implemented at flash_driver.c:1079 — and test the block protection bits directly.

Two facts make this the natural choice:

- FLASH_STATUS_BITS (0x3C) is already defined at the top of the same file and is the mask ns_setstatusbits() writes into the status register, so read and write stay self consistent.

- The Ameba side already does exactly this in os/se/ameba/rtl_security_api_wrapper_tz.c:99 (rtl_verify_flash_protect reads the status register and masks it with the same constant), so the two board back ends now behave identically.

It is also the semantically correct check. The demo is verifying whether the hardware is actually protecting the secure storage area; the status register is the hardware truth, whereas a driver-side cached value would only report the most recently requested setting.

## 4.2 The change

diff --git a/os/se/armino/bk_security_api_wrapper_tz.c b/os/se/armino/bk_security_api_wrapper_tz.c

--- a/os/se/armino/bk_security_api_wrapper_tz.c

+++ b/os/se/armino/bk_security_api_wrapper_tz.c

@@ -123,12 +123,12 @@ int bk_ss_flash_read(uint32_t address, uint32_t len, uint8_t *data, int en_displ

*/

int bk_verify_flash_protect(void)

{

-	flash_protect_type_t protect_type;

+	uint16_t status_reg;

-	protect_type = bk_flash_get_protect_type();

-	printf("Flash Protect Type: %d\n", protect_type);

+	status_reg = bk_flash_read_status_reg();

+	printf("Flash Status Register: 0x%04x\n", status_reg);

-	if (protect_type != FLASH_PROTECT_NONE) {

+	if ((status_reg & FLASH_STATUS_BITS) == FLASH_STATUS_BITS) {

return 1;

} else {

return 0;

The complete patch is committed alongside this document as os/0001-os-se-armino-fix-bk7239n-link-failure-on-undefined-b.patch and can be applied with git am.

## 4.3 Alternative considered

The declared-but-missing API could instead be implemented in the driver, keeping the wrapper as written:

/* os/board/bk7239n/src/middleware/driver/flash/flash_driver.c */

flash_protect_type_t bk_flash_get_protect_type(void)

{

return s_flash_runtime_protect_type;

}

This was rejected. s_flash_runtime_protect_type records the last protection type the driver was asked to apply, not the state of the hardware, so the demo would report a protection status it never actually verified. It also widens the change into the vendor driver for no benefit. If the public API is wanted for its own sake it should be added as a separate change, together with bk_flash_set_protect_type().

# 5. Verification

Reproduction and verification were both done with a full out-of-tree build of the affected configuration.

| Step | Result |
| --- | --- |
| Configure bk7239n/hello at master 5dad7bae | Configuration is Done! |
| Full make, unmodified master | FAILED — undefined reference to bk_flash_get_protect_type at pass2 link |
| Full make, with the fix applied | PASSED — LD: tinyara succeeded, build/output/bin/tinyara.axf produced |
| Number of remaining unresolved symbols | Zero — the link completes, so this was the only one |

After the successful link the local build stops in the board post-build step, in os/tools/mkbootparam.py and mkchecksum.py. Those are Python 2 scripts and the local container only has Python 3; the CI docker image (tizenrt/tizenrt:2.0.1) provides Python 2, so this is a local environment artefact and is unrelated to the defect.

# 6. Secondary Observation (not a build issue)

An automated review on PR #7312 flagged a runtime concern that was not addressed before merge. It does not affect the build and is recorded here only so it is not lost.

In apps/examples/board_specific_demo/secure_storage_demo/secure_storage.c the raw flash address is computed as address = (slot == 0) ? SLOT0_START_ADDR : SLOT1_START_ADDR + SLOT_SIZE * (slot - 1), and the demo accepts any slot index from 0 to 31. With the Armino constants from armino.h (SLOT1_START_ADDR = 0x2E000, SLOT_SIZE = 0x1000, SLOT_AREA_OFFSET = 0x2A000) slot 31 resolves to 0x4C000, and the area B address to 0x76000.

Step 8 of the demo calls BOARD_FLASH_ERASE() on that computed address, so a high slot index could erase outside the secure storage partition. The partition bounds for bk7239n were not confirmed as part of this analysis, so this should be validated separately; if confirmed, the accepted slot range should be clamped to the slots the partition actually holds.

# 7. Reference Locations

| Location | Relevance |
| --- | --- |
| os/se/armino/bk_security_api_wrapper_tz.c:124 | bk_verify_flash_protect() — the function that fails to link |
| os/se/armino/Make.defs:25 | SE_CSRCS += bk_security_api_wrapper_tz.c (gated on CONFIG_SE_ARMINO only) |
| os/board/bk7239n/src/include/driver/flash.h:131 | Declaration of bk_flash_get_protect_type() with no definition |
| os/board/bk7239n/src/include/driver/flash.h:124 | Declaration of bk_flash_set_protect_type() with no definition |
| .../middleware/driver/flash/flash_driver.c:190 | static s_flash_runtime_protect_type |
| .../middleware/driver/flash/flash_driver.c:620 | static flash_set_protect_type() |
| .../middleware/driver/flash/flash_driver.c:1079 | bk_flash_read_status_reg() — the API used by the fix |
| os/se/ameba/rtl_security_api_wrapper_tz.c:99 | rtl_verify_flash_protect() — the pattern the fix mirrors |
| build/configs/bk7239n/hello/defconfig:1071 | CONFIG_SE_ARMINO=y |
| .circleci/config.yml:225 | bk7239n_hello job definition |