How Commit dc535da Works in Flat Mode

Nested interrupt handling on rtl8721csm (Cortex-M33, ARMv8-M), explained in simple phases

| Item | Details |
| --- | --- |
| Commit | dc535da: "arch/arm : Fix Usage fault due to wrong handling of nested interrupts" |
| Pull request | Samsung/TizenRT #5397 (this document covers only this commit) |
| Board / CPU | rtl8721csm (Realtek AmebaD), Cortex-M33, ARMv8-M. TizenRT runs in the Non-secure world. |
| Build mode | Flat (CONFIG_BUILD_FLAT): hello, tc, nettest, ipv6, rdp, cxxtest, tls_test, security_hal_test_tz |
| Key settings | No FPU, no separate interrupt stack (CONFIG_ARCH_INTERRUPTSTACK=0), BASEPRI used, interrupts masked with cpsid i inside the handler |
| Main file | os/arch/arm/src/armv8-m/up_exception.S |

At a glance

In flat mode every task and every interrupt uses the same stack pointer (MSP). The main protections in this commit were made for protected mode, so in flat mode they are inactive or do nothing.

What really changes in flat mode: (1) interrupts are switched off at the very first instruction, (2) one extra word is saved for every interrupt, and (3) as a side effect, interrupt C code now runs with a correctly 8-byte-aligned stack.

Nested interrupts were already stack-safe in flat mode before this commit.

# 1. Terms you need

| Term | Plain meaning |
| --- | --- |
| Thread mode / Handler mode | Thread mode means a task is running. Handler mode means an interrupt is running. |
| MSP / PSP | Two stack pointer registers. In flat mode every task uses MSP. PSP is never used. |
| Hardware frame | 8 registers (R0-R3, R12, LR, PC, xPSR) that the CPU saves by itself when an interrupt comes. |
| Software frame | The remaining registers, saved by the TizenRT code exception_common. |
| EXC_RETURN | A special value the CPU puts in LR during an interrupt. It tells the CPU where to go back to. 0xFFFFFFB8 = back to a task, on MSP. 0xFFFFFFB0 = back to another interrupt that was interrupted (nested). |
| BASEPRI / PRIMASK | Interrupt masks. cpsid i switches all normal interrupts OFF. cpsie i switches them back ON. |

# 2. Why the commit exists

The real problem was in protected mode. There, user tasks use PSP, and after an interrupt MSP was left pointing inside a user task's stack (a "stale" MSP). If a second (nested) interrupt came at the wrong moment, the CPU saved its registers on top of that user stack. The stack got corrupted and the system later crashed with a UsageFault.

The commit adds two protections:

- A saved copy of PSP for nested interrupts.

- A small 512-byte spare stack for MSP, used just before returning to a user task.

In flat mode MSP always points to the running task's own stack, so this problem cannot happen. Most of the commit is therefore "sleeping" in flat mode. The phases below show exactly what still runs.

# 3. The 4 phases of one interrupt (flat mode)

## Phase 1: The interrupt arrives (done by the CPU, no code)

Task T is running on its own stack (MSP). An interrupt comes. The CPU does four things automatically:

- Pushes the 8 hardware-frame registers onto T's stack. It also keeps SP a multiple of 8.

- Switches to Handler mode.

- Puts 0xFFFFFFB8 in LR (meaning: later, return to a task on MSP).

- Jumps to exception_common.

T's stack (MSP)

+----------------------+

| T's own data         |

+----------------------+ <- T's SP before the interrupt

| Hardware frame (8)   |    xPSR, PC, LR, R12, R3, R2, R1, R0

+----------------------+ <- MSP now

| free space           |

Changed by the commit: nothing.

## Phase 2: Lock the door, then save the rest (entry code)

This is the entry code exactly as it is built for flat mode, with plain comments. Lines marked NEW or MOVED come from this commit.

mrs   r3, basepri       ; remember the current BASEPRI   <- MOVED to top

cpsid i                 ; switch all interrupts OFF      <- MOVED to top

tst   lr, #4            ; was the frame saved on PSP?

beq   1f                ;   flat mode: never PSP, so always jump

1:

mov   r2, sp            ; r2 = where the hardware frame is

add   r2, r2, #32       ; r2 = T's SP before the interrupt

mov   r1, #0            ; "PSP word" = 0 ...             <- NEW

cmp   lr, #0xFFFFFFB0   ; ... unless we interrupted another IRQ <- NEW

bne   10f               ;                                <- NEW

mrs   r1, psp           ; then "PSP word" = PSP          <- NEW

10:

str   r1, [sp, #-4]!    ; save the PSP word (1 word)     <- NEW

push  {r2-r11, lr}      ; save old SP, BASEPRI, R4-R11, EXC_RETURN

Two things change:

- Interrupts are switched off at the very first step. Before the commit this happened about 7 instructions later, after the saving. So the gap where another interrupt can sneak in drops from about 7 instructions to 1.

- One extra word is saved: the "PSP word". For a normal interrupt it is always 0 in flat mode. It only holds a value when this interrupt has interrupted another interrupt (LR = 0xFFFFFFB0), and even then flat mode never uses it.

After Phase 2, the full saved context is 20 words (80 bytes):

+----------------------+ <- T's SP before the interrupt

| Hardware frame (8)   |  index 12..19: R0,R1,R2,R3,R12,LR,PC,xPSR

| PSP word = 0         |  index 11  <- NEW (R0..xPSR moved up by one)

| EXC_RETURN           |  index 10

| R11 .. R4            |  index 2..9

| BASEPRI              |  index 1

| old SP               |  index 0

+----------------------+ <- MSP = "regs" pointer

The same layout as a table (this is also how it is stored in each task's TCB, tcb->xcp.regs):

| Index | Saved value | Saved by | Index before commit |
| --- | --- | --- | --- |
| 0 | SP before the interrupt (REG_R13) | software | 0 |
| 1 | BASEPRI (REG_BASEPRI) | software | 1 |
| 2-9 | R4-R11 | software | 2-9 |
| 10 | EXC_RETURN (REG_EXC_RETURN) | software | 10 |
| 11 | PSP word (REG_PSP), NEW | software | (did not exist) |
| 12-19 | R0, R1, R2, R3, R12, LR, PC, xPSR | hardware (CPU) | 11-18 |

## Phase 3: Handle the interrupt (C code)

mov   r1, sp            ; r1 = pointer to the saved registers

mrs   r0, ipsr          ; r0 = interrupt number          <- MOVED here

bl    up_doirq          ; run the ISR; returns which registers to restore

- up_doirq sets current_regs = regs and runs the driver ISR. There is no separate interrupt stack, so all of this runs on T's stack, just below the saved frame.

- If the ISR wakes a higher-priority task N, the scheduler copies T's 20 saved words into T's TCB and points current_regs at N's saved words.

- up_doirq returns that pointer: either T's own frame, or N's saved registers.

Side benefit in flat mode: stack alignment.

The ARM C calling rules (AAPCS) expect SP to be a multiple of 8 at every function call. The CPU leaves SP 8-byte aligned after its hardware frame, so the size of the software frame decides the final alignment:

|  | Before commit | After commit |
| --- | --- | --- |
| Software frame size | 11 words = 44 bytes | 12 words = 48 bytes |
| SP when up_doirq and the ISR run | Off by 4 (not a multiple of 8) | Multiple of 8 (correct) |

So in flat mode the commit fixes the stack alignment by accident.

## Phase 4: Restore registers and return (exit code)

There are two roads, then a common ending.

### Road A: same task T (no task switch)

ldm   r1!, {r2-r11, lr} ; take back T's registers from the stack

ldm   r1!, {r2}         ; take back the PSP word         <- NEW

### Road B: new task N (task switch)

add   r1, r0, #48       ; find N's hardware part inside N's TCB

ldm   r1!, {r4-r11}     ; read those 8 words

ldr   r1, [r0]          ; r1 = N's stack pointer

stmdb r1!, {r4-r11}     ; write the 8 words onto N's stack

ldm   r0!, {r2-r11, lr} ; load N's BASEPRI, R4-R11, EXC_RETURN

ldm   r0!, {r2}         ; read N's PSP word              <- NEW

In Road B, the 8 hardware-frame words of N are copied from N's TCB back onto N's own stack, so the CPU can pop them in the last step.

### Common ending

cmp   lr, #0xFFFFFFB0   ; going back into another interrupt? <- NEW

bne   4f                ;                                <- NEW

msr   psp, r2           ; put PSP back (no effect in flat mode) <- NEW

4:

msr   msp, r1           ; MSP = the task's hardware frame

msr   basepri, r3       ; restore BASEPRI

cpsie i                 ; switch interrupts back ON

bx    lr                ; CPU pops the 8 registers -> task continues

When the CPU executes bx lr with the special EXC_RETURN value, it pops the 8 hardware-frame registers from MSP and the task continues exactly where it was.

In protected mode, MSP would now be pointed at the 512-byte spare stack. That code is not built in flat mode, so MSP simply goes back to the task's own stack.

# 4. What if a second interrupt comes in the middle? (nested, flat mode)

Two tiny gaps remain in which another, higher-priority interrupt can come in:

- Gap 1: right at the start, before cpsid i. This is now about 1 instruction.

- Gap 2: right at the end, after cpsie i and before bx lr. This is 1 instruction, the same as before.

If a higher-priority interrupt W comes in either gap while interrupt Z is being handled:

T's stack (MSP)

+----------------------+

| T's data             |

| Z: hardware frame    |  <- first interrupt

+----------------------+  <- MSP points here (valid, T's free space)

| W: hardware frame    |  <- second interrupt, LR = 0xFFFFFFB0

| W: software frame    |     (its PSP word = PSP; unused in flat mode)

| W: ISR stack use     |

+----------------------+

- W runs completely and returns.

- Z then continues from where it stopped.

This is safe in flat mode, because MSP always points to free space on the same task's stack. It was already safe before this commit.

One limitation the commit does not change: suppose W wakes a higher-priority task. TizenRT switches to that task right away, even though Z has not executed its last instruction. Z stays "active" until T runs again. Until then, interrupts at Z's priority or lower must wait. Everything continues correctly once T runs again.

# 5. What the commit really does in flat mode on rtl8721csm

| Change | Runs in flat? | Effect in flat mode |
| --- | --- | --- |
| Interrupts off at the 1st instruction | Yes | The gap at the start shrinks from about 7 instructions to 1. Small gain. |
| Extra "PSP word" in the frame | Yes | It is 0, or an unused PSP value. Adds 4 bytes per interrupt and 4 bytes per task TCB. |
| Frame grows 11 to 12 words | Yes | Good side effect: C code now runs with an 8-byte-aligned stack. |
| Restore PSP on a nested return | Yes (nested only) | No effect, because PSP is unused. |
| Move MSP to the 512-byte spare stack | No (protected only) | Not built in flat mode. |
| 512-byte g_nestedirqstk buffer | Built, never used | Probably removed by the linker (--gc-sections). |
| New Kconfig option | Must be set | CONFIG_ARCH_NESTED_IRQ_STACK_SIZE must not be 0, or the build stops with #error. Keep 512. |

# 6. Practical notes for flat mode

- Keep CONFIG_ARCH_NESTED_IRQ_STACK_SIZE=512 in your defconfig. The commit added it to all rtl8721csm defconfigs.

- Register positions shifted by one. R0 is now index 12 (it was 11), and xPSR is 19 (it was 18). Anything that reads tcb->xcp.regs by raw numbers must use the REG_xxx macros. No script in os/tools reads it by fixed index.

- Cost: about 8 extra instructions per interrupt, which is negligible.

- This commit does not fix flat-mode UsageFaults by itself. Its real fixes are for protected mode. If you see a UsageFault in flat mode, check CFSR/UFSR, PC, LR and EXC_RETURN in the crash log.

## Files changed by the commit

| File | What changed |
| --- | --- |
| os/arch/arm/src/armv8-m/up_exception.S | Entry and exit code of every interrupt (all 4 phases). |
| os/arch/arm/include/armv8-m/irq_cmnvector.h | Saved register layout: new REG_PSP word at index 11. |
| os/arch/arm/src/armv8-m/exc_return.h | EXC_RETURN_HANDLER set to 0xFFFFFFB0. |
| os/arch/Kconfig | New option CONFIG_ARCH_NESTED_IRQ_STACK_SIZE (default 512). |
| build/configs/rtl8721csm/*/defconfig | CONFIG_ARCH_NESTED_IRQ_STACK_SIZE=512 added to all 9 configs. |

# 7. Summary

## Key points

- In flat mode all tasks and interrupts use MSP. The commit's main fixes (the PSP save/restore and the spare MSP stack) are aimed at protected mode; in flat mode they are inactive or do nothing.

- What really changes in flat mode: interrupts are switched off at the first instruction (smaller gap), one extra saved word (always 0 for normal interrupts) and, as a result, an 8-byte-aligned stack for interrupt C code.

- Nested interrupts in flat mode were already stack-safe. The limitation "a nested interrupt that switches tasks keeps the first interrupt active" is still there.

## Next steps

- Keep the Kconfig value at 512, and use the REG_xxx macros in any tool that reads saved registers.

- Run the rtl8721csm flat stress test. If a fault appears, capture CFSR/UFSR, PC, LR and EXC_RETURN to find the real root cause.

- If flat mode should be leaner, the protected-only parts can be wrapped in #ifdef CONFIG_BUILD_PROTECTED. But keep the software frame a multiple of 8 bytes (or align SP before bl up_doirq), or the alignment benefit is lost.