# KASan for TizenRT

Phase 3 Readiness Report

Verification completed through Phase 2, and what must be checked before Phase 3 begins

| Field | Value |
| --- | --- |
| Revision | 1.0 |
| Branch | adbtrr_work10_kasan_plan |
| Phases complete | Phase 1 (runtime), Phase 2 (allocator hooks) |
| Head commit | 082fa753 |
| Bring-up target | rtl8730e / loadable_ext_ddr |
| Build model | CONFIG_BUILD_PROTECTED, APP_BINARY_SEPARATION, SMP x2, ARMv7-A |
| Verification to date | Host only. Nothing has run on hardware. |

# 1. Verification completed through Phase 2

Scope of this section.  Every figure below was measured by re-running the suites, not carried over from earlier notes. All of it is host-side. The runtime is target independent apart from machine word size, so the suites are run at 32 bit width to match ARM32, where a shadow granule is 4 bytes.

## 1.1 Results at a glance

| Suite | What it checks | Result | Catches |
| --- | --- | --- | --- |
| A | Runtime behaviour: carve arithmetic, poison and unpoison, mask paths, registration refusals, unregister, the gate, end to end detection, dump safety | 8 / 8 pass | Shadow map logic |
| B | Unaligned granule sweep: every offset and length across a block boundary | 0 missed of 128 | Detection blind spots |
| C | Phase 2 hook sequence, transcribed call for call from the patched allocator | 8 / 8 pass | Hook ordering, off by one |
| D | Build matrix under -Werror: 2 word widths x 3 optimisation levels x 3 configurations | 18 / 18 pass | Config permutations |
| E | ABI completeness against genuinely instrumented objects | 39 defined, 9 required, 0 unresolved | Missing entry points |
| F1 | Syntax against the real defconfig: 6 memory manager files x kernel and user pass x KASan off and on | 24 / 24 pass | Target specific breakage |
| F2 | Kernel files compared against the unmodified baseline | No new diagnostics | Regressions in edited files |
| G | Build model split, proved by preprocessing | user 0 calls, kernel 1 call | Wrong heap registered |
| H | Cost when KASan is disabled | 2 instructions | Overhead when off |

## 1.2 What each suite established

### Suite A — runtime behaviour

Eight tests drive the shadow directly. The carve arithmetic was measured rather than assumed: 2060 bytes of shadow for a 64 KB region, which is 1/31.8, confirming the 3.125 percent figure in the plan. Registration refusals cover a region too small to carve, a NULL address, and exhaustion of the region table, all of which must decline rather than corrupt memory.

### Suite B — the unaligned granule sweep

This suite exists because it found a real defect. The upstream NuttX poison test derives the granule count from the access size alone, ignoring where the access starts within its first granule. An unaligned access straddling a granule boundary was therefore checked against the first granule only, and that blind spot sits exactly where an overflow leaves a block.

| Measurement | Before the fix | After the fix |
| --- | --- | --- |
| Missed detections across every offset and length at a block boundary | 28 | 0 |

The correction is carried in os/mm/kasan/generic.c and is worth offering back upstream.

### Suite C — the Phase 2 hook sequence

The real memory manager cannot be compiled on a host, so this suite performs the exact sequence of KASan calls each patched function makes, in the same order and with the same arguments, and then asserts the resulting shadow state. It verifies the property the hooks exist to maintain: free memory is closed, and only the user area of a live allocation is open.

| Property | Why it matters |
| --- | --- |
| Node header stays closed around an open block | This is what turns a write below the returned pointer into a reported underflow |
| Adjacent blocks stay separated | Running off the end of one block must be reported, not land silently in the next |
| A shrink closes the released tail | Otherwise realloc quietly leaves freed memory readable |
| A grow opens before the copy, not after | memcpy lives in lib/libc and is instrumented; copying into still poisoned memory would report a false use after free from inside realloc |
| memalign opens only the aligned chunk | The remainders split off either side go back to the free list and must stay closed |
| An overlapping region is refused | Two shadows for one address range would leave the second permanently stale |

### Suite E — ABI completeness

Rather than trusting documentation, code was compiled with the exact flag set this port uses and the required symbols read out of the object file. The runtime defines a superset, and an instrumented object links against it with nothing unresolved.

__asan_handle_no_return

__asan_load1_noabort   __asan_load2_noabort   __asan_load4_noabort

__asan_loadN_noabort

__asan_store1_noabort  __asan_store2_noabort  __asan_store4_noabort

__asan_storeN_noabort

### Suite G — the build model split

On a protected build os/mm is compiled twice. Preprocessing both passes proves the split is real rather than intended: the user pass keeps only the prototype and emits no call, while the kernel pass emits one call guarded by the g_kmmheap bound test.

| Pass | kasan_register calls | Meaning |
| --- | --- | --- |
| libumm (user, no __KERNEL__) | 0 | User heap never registered |
| libkmm (kernel, __KERNEL__) | 1, behind the g_kmmheap test | Kernel heap only; app heaps excluded |

## 1.3 What has NOT been verified

Read this before treating Phase 2 as done.  Nothing in this project has run on hardware, or even been compiled by an ARM toolchain. All verification so far is host side. The suites prove the shadow logic and the hook ordering are correct; they cannot prove the reordered mm_addregion produces a working heap on a real board, and they cannot produce a false positive rate. Both of those arrive only in Phase 3.

Specifically unproven:

- That the cross toolchain supports -fsanitize=kernel-address at all. This is gate A1 below and blocks everything.

- That the instrumented image fits the flash partition. See section 2.2.

- That the board boots with a heap whose tail has been carved away.

- The real false positive rate under Wi-Fi, filesystem and binary manager traffic.

- The real cost in image size and throughput on this silicon.

# 2. What must be checked before Phase 3

Phase 3 adds the compiler flags to the board Make.defs, which is the first step that puts KASan on hardware. The checks below are ordered so that a failure is attributable: each one removes a possible cause of the next step failing.

## 2.1 Blocking gates

These are pass or stop.  If A1 fails the feature cannot be built at all, and the rest of the plan does not apply.

### A1. The cross toolchain supports kernel-address sanitizing

echo 'int f(int *p){ return *p; }' | arm-none-eabi-gcc \

-fsanitize=kernel-address \

--param asan-instrumentation-with-call-threshold=0 \

-S -x c - -o -

# Required in the output:   bl  __asan_load4_noabort

A bare metal GCC can be configured without sanitizer support. If the call does not appear, stop: no amount of TizenRT side work will help. Status: not yet run, no ARM toolchain was available during development.

### A2. The toolchain accepts the full parameter set

arm-none-eabi-gcc -fsanitize=kernel-address \

--param asan-stack=0 \

--param asan-globals=0 \

--param asan-instrumentation-with-call-threshold=0 \

-c -x c /dev/null -o /dev/null

Older GCC spells some parameters differently. Confirm all three are accepted before wiring them into Make.defs.

### A3. A clean baseline build, before any KASan flag

Build rtl8730e/loadable_ext_ddr from the current branch head with CONFIG_MM_KASAN left off, and record that it succeeds. Phases 1 and 2 are dormant when the option is off, so this build must be identical in behaviour to the pre-KASan tree. Record the kernel image size here; section 2.2 needs it. Without this baseline a Phase 3 failure cannot be attributed to the flags rather than to the hooks.

## 2.2 Budget: the check most likely to stop Phase 3

The kernel image is flashed into a fixed partition on this board:

| Partition | Size | Note |
| --- | --- | --- |
| kernel | 2868 KB | The instrumented image must fit here |
| common | 8476 KB | Common binary |
| app1 | 4780 KB | Application |
| ss | 400 KB | Secure storage |

Do this arithmetic before writing a single flag.  Instrumentation is expected to grow text by 30 to 50 percent. Take the kernel image size from gate A3, add 50 percent, and compare against 2868 KB. If it does not fit there are three levers, in order of preference: instrument a subset of directories rather than the whole image, set CONFIG_MM_KASAN_DISABLE_READS_CHECK to instrument writes only, or enlarge the kernel partition. os/tools/check_partition.py and check_package_size.py already exist to validate the result.

Two further budgets, both comfortable on this configuration but worth recording:

| Budget | Cost | Headroom on this board |
| --- | --- | --- |
| RAM for the shadow | About 2 MB | 64 MB region, so about 3.1 percent |
| Thread stack during a report | A 72 byte line buffer plus lowsyslog | Reports run on the faulting thread stack |

## 2.3 Pre-existing issues that affect Phase 3

Both were found while preparing this report. Neither is caused by the KASan work, and neither has been changed.

### D1. CONFIG_FRAME_POINTER does the opposite of its name on this board

build/configs/rtl8730e/Make.defs adds -fomit-frame-pointer when CONFIG_FRAME_POINTER is enabled. Every other board in the tree, and the Kconfig help text itself, uses -fno-omit-frame-pointer.

| Board | Flag added when CONFIG_FRAME_POINTER=y |
| --- | --- |
| rtl8730e | -fomit-frame-pointer (does the opposite) |
| rtl8721csm | -fomit-frame-pointer (same copy of the bug) |
| artik053 | -fno-omit-frame-pointer (correct) |
| cy4390x | -fno-omit-frame-pointer (correct) |
| sidk_s5jt200 | -fno-omit-frame-pointer (correct) |

Consequence for KASan.  The target defconfig does not set CONFIG_FRAME_POINTER, so KASan reports will carry the faulting address and the calling address but no backtrace. Enabling the option to improve reports will not work until this line is corrected, and the failure is silent: dump_stack() compiles and runs but walks frames that were never pushed. Decide before Phase 3 whether backtraces are wanted; if so, fix this line first and treat it as a separate change.

### D2. -Werror is narrower than assumed

-Werror applies on this board only when QUICKBUILD=y. The default build does not use it, so the risk that new instrumentation warnings break the build is lower than the plan document states. It is still worth running one QUICKBUILD=y build during Phase 3 to catch them deliberately rather than discovering them later in CI.

## 2.4 Decisions to make before writing the flags

| Decision | Recommendation |
| --- | --- |
| Which directories to instrument first | Exclude the vendor SDK trees, os/board/rtl8730e/src and os/board/bk7239n/src/components, on the first build. Instrument TizenRT authored code, get a clean boot, then widen. |
| Reads and writes, or writes only | If the size budget in 2.2 is tight, start with CONFIG_MM_KASAN_DISABLE_READS_CHECK. Most corruption bugs are writes, and it roughly halves both the size and the speed cost. |
| Panic or continue on a report | Keep the default, panic. The crash dump is taken at the faulting instruction, which is the whole point. Switch to continue only when collecting several faults in one boot. |
| CONFIG_MM_KASAN_REGIONS | Leave at 8. This configuration registers exactly one region: KMM_REGIONS is 1 and KMM_NHEAPS is 1. |
| Backtraces | See D1. Either accept caller address only, or fix the board Make.defs first. |

## 2.5 Constraints already confirmed, no action needed

| Constraint | Status |
| --- | --- |
| mm_extend() is incompatible with the shadow placement | Confirmed unreachable. A tree wide search finds no caller of kmm_extend or umm_extend, and mm_sbrk is CONFIG_BUILD_KERNEL only while this board is CONFIG_BUILD_PROTECTED. A comment at the site warns anyone who wires one up. |
| App heaps must not be registered | Handled. binfmt runs in the kernel and builds each app heap, so the build guard alone was not enough; mm_addregion also tests that the heap lies within g_kmmheap. |
| The memory manager must not instrument itself | Handled by directory. One CFLAGS serves all of os/mm, so the -fno-sanitize in kasan/Make.defs excludes the whole directory, including the free path and the heap walkers inside it. |
| In tree heap walkers outside os/mm | Handled. mem_leak_checker.c and wd_dbg.c suspend checking across their walks. CONFIG_MEM_LEAK_CHECKER is enabled on this target, so this was not optional. |
| Vendor binaries cannot be instrumented | Accepted limitation. Their accesses are invisible, which produces false negatives, never false positives. |

## 2.6 Order of work

- 1. Run gate A1. Stop here if it fails.

- 2. Run gate A2, then build the A3 baseline and record the kernel image size.

- 3. Do the partition arithmetic in 2.2. If it does not fit, decide which lever to pull before writing any flag.

- 4. Settle the decisions in 2.4, particularly the vendor exclusions and the backtrace question.

- 5. Only then add the flags to build/configs/rtl8730e/Make.defs and start Phase 3.

## 2.7 What Phase 3 must then demonstrate

In this order, because each failure has a different cause:

| Step | Check | A failure here means |
| --- | --- | --- |
| 1 | The instrumented image builds | Flag or warning fallout, or a vendor tree that cannot be instrumented |
| 2 | It fits the kernel partition | The budget in 2.2 was wrong; pull a lever |
| 3 | It boots to the TASH prompt | The gate variable or kasan_init_early ordering is wrong |
| 4 | heapinfo shows free heap reduced by about 3 percent | kasan_register did not carve, or the heap was rebuilt wrongly |
| 5 | Idle 60 seconds, then exercise Wi-Fi, filesystem and binary manager with no report | A false positive to root cause, not a bug found |
| 6 | leak check runs to completion | The Phase 2 bracketing is wrong or incomplete |

The one that matters is step 5.  Steps 1 to 4 prove the feature is present. Step 5 proves it is usable. Budget most of Phase 3 for eliminating false positives, not for confirming that true positives fire, which is Phase 5.