# Scenario Based Test Cases - Priority Inheritance and Semaphore Recovery

Document version : 1.0 Applies to       : TizenRT kernel (os/kernel/semaphore, os/kernel/pthread, os/kernel/task, os/kernel/binary_manager)

## 1. Purpose and Scope

This document defines scenario based test cases for two closely coupled areas of the TizenRT kernel:

| Part | Area | Primary source |
| --- | --- | --- |
| A | Priority inheritance (PI) on semaphores and pthread mutexes | os/kernel/semaphore/sem_holder.c, sem_wait.c, sem_post.c, sem_setprotocol.c |
| B | Semaphore recovery for mutex usage and for signalling usage / holder accounting | os/kernel/semaphore/sem_recover.c, sem_holder.c (sem_release_all), sem_waitirq.c, sem_destroy.c, os/kernel/task/task_recover.c, os/kernel/pthread/pthread_mutexinconsistent.c, os/kernel/binary_manager/binary_manager_recovery.c |

The two parts are separated only for readability. They share the same holder book-keeping structures (struct semholder_s, tcb->holdsem, sem->hhead / sem->holder), so several scenarios deliberately exercise both at once.

Out of scope: SMP specific spinlock behaviour (sem_holder's data is protected by enter_critical_section(); SMP stress is covered by a separate scheduler document), named semaphores (sem_open/sem_close) and POSIX message queues.

## 2. Behaviour Under Test - Reference Model

The expected results in every scenario are derived from the following model of the current implementation. Any scenario whose observed behaviour differs from this model is a FAIL.

### 2.1 Semaphore flavours

sem_init() and pthread_mutex_init() stamp sem->flags, and the flavour decides how much book-keeping the semaphore gets:

| Flavour | How it is created | Flags | Holder recorded? | PI active? |
| --- | --- | --- | --- | --- |
| Counting / generic | sem_init(sem, 0, N) with N > 0 | FLAGS_INITIALIZED | Yes | Yes |
| Signalling | sem_init(sem, 0, 0) - sem_init() sets FLAGS_SIGSEM automatically when the initial value is 0 | FLAGS_INITIALIZED ¦ FLAGS_SIGSEM | No (sem_addholder_tcb() and sem_releaseholder() return early) | Effectively no - there is never a holder to boost |
| Mutex | pthread_mutex_init() - sem_init(&mutex->sem, pshared, 1) then flags ¦= FLAGS_SEM_MUTEX | FLAGS_INITIALIZED ¦ FLAGS_SEM_MUTEX | Yes | Per attr->proto (PTHREAD_PRIO_INHERIT by default) |
| PI disabled | sem_setprotocol(sem, SEM_PRIO_NONE) | ... ¦ PRIOINHERIT_FLAGS_DISABLE | Only when CONFIG_BINMGR_RECOVERY=y (holders are still needed for fault recovery); otherwise sem_destroyholder() drops them immediately | No |
| Condvar semaphore | COND_SEM_INITIALIZER() | FLAGS_INITIALIZED ¦ FLAGS_SIGSEM ¦ PRIOINHERIT_FLAGS_DISABLE | No | No |

FLAGS_SEM_MUTEX additionally arms the invariant semcount < 2 at all times. It is asserted in sem_wait(), sem_trywait(), sem_post(), sem_waitirq(), sem_recover(), binary_manager_recover_tcb() and binary_manager_release_binary_sem(). Any scenario that can drive a mutex-flagged semaphore to semcount >= 2 is a FAIL even if no assert build is used.

### 2.2 Priority inheritance model

1.  Boost - sem_wait() finds semcount <= 0, decrements it, records rtcb->waitsem, takes sched_lock(), and - only when PRIOINHERIT_FLAGS_DISABLE is clear - calls sem_boostpriority(). sem_boostholderprio() raises each holder whose sched_priority is strictly lower than the waiter's, via sched_setpriority(). base_priority is never touched by boosting.

2.  Restore - sem_post() calls sem_releaseholder() (decrements the holder's counts), increments semcount, and sem_unblock_task() hands the count to the head of the prioritised g_waitingforsemaphore list whose waitsem matches. Then sem_restorebaseprio(stcb, htcb, sem) runs:

•  from a task: two passes - sem_restoreholderprioA (every holder except the running task) then sem_restoreholderprioB (the running task last, so that it is not de-scheduled mid-operation);

•  from an ISR: a single pass, sem_restorebaseprio_irq().

3.  Target priority - sem_restoreholderprio() computes hpriority = max(htcb->boost_priority, htcb->base_priority) and then walks htcb->holdsem. For each still-held entry (counts > 0) it takes sem_findhighestwaiter(hpholder->sem, exclude) and raises hpriority to the highest waiter found. Entries with counts <= 0 are skipped - that entry is the semaphore currently being released, and DEBUGASSERT(hpholder->sem == sem) documents it.

4.  Exclusion - the task that is leaving the wait (received the count, or was cancelled) is passed as exclude and must be ignored by sem_findhighestwaiter(), because it can still be linked on g_waitingforsemaphore while priorities are recomputed.

5.  PI-disabled semaphores never contribute - sem_findhighestwaiter() returns NULL for a semaphore carrying PRIOINHERIT_FLAGS_DISABLE, even though such a semaphore may still appear on tcb->holdsem when CONFIG_BINMGR_RECOVERY=y.

6.  Holder removal - after restoration, sem_restorebaseprio() frees the holder container when its counts <= 0.

7.  Cancellation - sem_canceled() (from sem_waitirq(), sem_recover() and binary_manager_recover_tcb()) runs the same restore pass over all holders, again with the leaving task as exclude.

8.  Explicit reprioritisation wins - sched_reprioritize() sets base_priority and clears boost_priority, discarding inheritance history.

### 2.3 Recovery model

| Trigger | Entry point | Actions |
| --- | --- | --- |
| task_delete(), pthread_cancel(), thread exit | task_recover() -> sem_recover() | If the victim is in TSTATE_WAIT_SEM: sem_canceled(), semcount++, waitsem = NULL, state deliberately left as TSTATE_WAIT_SEM (the TCB is still linked in the wait list). Then sem_release_all() for every semaphore the victim holds. |
| sem_release_all() | sem_holder.c | For each entry of htcb->holdsem: sem_freeholder(), semcount++, and if semcount <= 0 hand the count to the first matching waiter (sem_addholder_tcb(), waitsem = NULL, up_unblock_task()). |
| Signal / timeout during wait | sem_waitirq() | sem_canceled(), semcount++, waitsem = NULL, pterrno = EINTR/ETIMEDOUT, up_unblock_task(). sem_wait() then returns ERROR without touching the count again. |
| pthread terminated holding a mutex | pthread_mutex_inconsistent() | Each mutex on tcb->mhead gets _PTHREAD_MFLAGS_INCONSISTENT and is posted; the next pthread_mutex_lock() returns EOWNERDEAD. |
| sem_destroy() | sem_destroy.c | semcount reset to 1 when non-negative, sem_destroyholder() frees every stranded holder (also unlinking them from each holder task's holdsem list), unregister from g_sem_list for non-signalling semaphores under CONFIG_BINMGR_RECOVERY, flags = 0. |
| User binary fault | binary_manager_deactivate_binary() | binary_manager_recover_tcb() per thread (semaphore / mqueue / watchdog), then binary_manager_release_binary_sem() walks g_sem_list and releases kernel semaphores held by that binary. |

Note on registration: sem_init() registers a semaphore into g_sem_list only when semcount != 0 and it lives in the kernel region. Signalling semaphores (initial value 0) are therefore never registered and are not covered by binary_manager_release_binary_sem().

## 3. Build Configuration Matrix

Every scenario names the configuration it needs. CFG-1 is the default reference configuration; run the full suite at least on CFG-1 and CFG-2.

| Id | PRIORITY_INHERITANCE | SEM_PREALLOCHOLDERS | BINMGR_RECOVERY | DEBUG / asserts | SEMAPHORE_HISTORY | SEM_PHDEBUG |
| --- | --- | --- | --- | --- | --- | --- |
| CFG-1 | y | 16 | n | y | y | y |
| CFG-2 | y | 0 (built-in single holder) | n | y | y | y |
| CFG-3 | y | 16 | y | y | y | y |
| CFG-4 | n | n/a | n | y | y | n |
| CFG-5 | y | 2 (holder starvation) | n | y | y | y |

CONFIG_SEM_PHDEBUG exposes sem_enumholders() / sem_nfreeholders(), which the test harness uses to observe holder containers without depending on kernel internals in the test code.

## 4. Test Harness, Notation and Instrumentation

### 4.1 Placement

The scenarios are written to be implemented under apps/examples/testcase/le_tc/kernel/ (tc_semaphore.c, itc_semaphore.c, tc_pthread.c), alongside the existing kernel test cases, and driven by kernel_tc_main.c.

### 4.2 Actor naming and priorities

| Symbol | Priority | Role |
| --- | --- | --- |
| T_L | 100 | Low priority task - typically the initial semaphore holder |
| T_M | 120 | Medium priority task - the "inverter", CPU bound, never touches the semaphore |
| T_H | 140 | High priority task - the blocked waiter |
| T_X | 160 | Extra-high priority task - second waiter, used for re-boost checks |
| ISR | - | Interrupt context (a watchdog/timer callback is sufficient) |

SCHED_FIFO is used everywhere unless the scenario says otherwise, so that a scenario never depends on round-robin timeslice expiry.

### 4.3 Observation points

| What | How |
| --- | --- |
| Effective priority | sched_getparam(pid, &param) -> param.sched_priority |
| Base priority | tcb->base_priority via a debug accessor, or inferred: after full restore, effective priority must equal the value set by the last sched_setparam() |
| semcount | sem_getvalue(sem, &val) |
| Holder list | sem_enumholders(sem) (CFG with SEM_PHDEBUG), free container count via sem_nfreeholders() |
| Wait/acquire/release ordering | CONFIG_SEMAPHORE_HISTORY ring (SEM_INIT, SEM_WAITING, SEM_ACQUIRE, SEM_RELEASE, SEM_DESTROY) |
| Task state | ps / tcb->task_state - used by recovery scenarios that must confirm TSTATE_WAIT_SEM is retained |
| Execution order | Each actor appends a character to a shared trace buffer; the scenario compares the buffer to an expected string |

### 4.4 Result notation used in the tables

prio(T) effective priority, base(T) base priority, count(S) value of S, holders(S) set of holder TCBs with their counts, [trace] the expected execution trace string.

### 4.5 Global pass rules (apply to every scenario)

| Id | Rule |
| --- | --- |
| G-1 | No assert, DEBUGASSERT, crash or watchdog reset during or after the scenario |
| G-2 | On completion, every participating task's effective priority equals its base priority |
| G-3 | On completion, count(S) equals the value the sequence arithmetic predicts, and never exceeds SEM_VALUE_MAX |
| G-4 | On completion, holders(S) is empty for every semaphore that has been fully released |
| G-5 | sem_nfreeholders() returns to its pre-scenario value (no holder container leak) |
| G-6 | For any FLAGS_SEM_MUTEX semaphore, count(S) < 2 at every observation point |
| G-7 | No task is left blocked after the scenario's terminating condition |

## 5. Part A - Priority Inheritance Scenarios

### PI-01 - Basic boost and restore on a binary semaphore

| Field | Value |
| --- | --- |
| Objective | The canonical boost/restore cycle: a low priority holder is lifted to the waiter's priority and returns to base on post. |
| Config | CFG-1, CFG-2 |
| Semaphore | S = sem_init(&S, 0, 1) (counting flavour, PI enabled) |
| Actors | T_L (100), T_H (140) |

Steps and expected results:

| # | Action | Expected state |
| --- | --- | --- |
| 1 | T_L : sem_wait(S) | count(S) = 0, holders(S) = {T_L:1}, prio(T_L) = 100 |
| 2 | T_H starts and calls sem_wait(S) | count(S) = -1, T_H in TSTATE_WAIT_SEM, T_H->waitsem = S, prio(T_L) = 140, base(T_L) = 100 |
| 3 | T_L : sem_post(S) | count(S) = 0, T_H unblocked and becomes holder, holders(S) = {T_H:1}, prio(T_L) = 100 |
| 4 | T_H : sem_post(S) | count(S) = 1, holders(S) empty |

Pass criteria: G-1..G-7; the boost in step 2 is visible before T_L posts (sampled by T_H's prologue is not possible - sample it from a third observer task at priority 160 that only reads sched_getparam(T_L)), and prio(T_L) drops in the same sem_post() call in step 3. Defect signature caught: missing sem_boostpriority() call, or restore leaving T_L permanently at 140. Code reference: sem_wait.c PI block, sem_holder.c:sem_boostholderprio(), sem_holder.c:sem_restorebaseprio_task().

### PI-02 - No boost when the waiter is not higher priority

| Field | Value |
| --- | --- |
| Objective | sem_boostholderprio() must apply strictly rtcb->sched_priority > htcb->sched_priority. |
| Config | CFG-1 |
| Actors | T_H (140) holds, T_L (100) waits, and a second run with an equal-priority waiter (140) |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | T_H : sem_wait(S) | count(S) = 0, prio(T_H) = 140 |
| 2 | T_L : sem_wait(S) blocks | count(S) = -1, prio(T_H) = 140 (unchanged) |
| 3 | Repeat with waiter at 140 | prio(T_H) = 140 (unchanged) - no self-inflicted sched_setpriority() and therefore no re-queueing of the holder behind its equals |

Pass criteria: prio(T_H) never changes; the holder is not moved to the tail of its ready-to-run priority group (verified by execution trace order of two equal-priority runnable tasks). Defect signature: a >= comparison in sem_boostholderprio() causing a spurious sched_setpriority() and a yield-like reordering.

### PI-03 - Successive boosts by increasingly higher waiters

| Field | Value |
| --- | --- |
| Objective | The holder tracks the maximum waiter priority as new waiters arrive. |
| Config | CFG-1 |
| Actors | T_L (100) holds, T_H (140), T_X (160) |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | T_L : sem_wait(S) | holders(S) = {T_L:1} |
| 2 | T_H : sem_wait(S) blocks | prio(T_L) = 140, count(S) = -1 |
| 3 | T_X : sem_wait(S) blocks | prio(T_L) = 160, count(S) = -2, wait list order is T_X, T_H |
| 4 | T_L : sem_post(S) | count handed to T_X (highest waiter), count(S) = -1, prio(T_L) restored to 100 - not to 140: T_H still waits, but T_L no longer holds a count (its holder entry reached 0 and is skipped, then freed) |
| 5 | T_X : sem_post(S) | count handed to T_H, prio(T_X) = 160 (never boosted), count(S) = 0 |
| 6 | T_H : sem_post(S) | count(S) = 1, holders empty |

Pass criteria: step 4 restores T_L fully to 100. A restore to 140 means the counts <= 0 skip in sem_restoreholderprio() is missing and the released holder is still being boosted by the leftover waiter. Code reference: sem_holder.c:sem_restoreholderprio() - the hpholder->counts <= 0 branch with DEBUGASSERT(hpholder->sem == sem).

### PI-04 - Holder of two semaphores restores to the maximum of remaining waiters

| Field | Value |
| --- | --- |
| Objective | sem_restoreholderprio() must scan the whole htcb->holdsem list, not just the semaphore being posted. |
| Config | CFG-1 |
| Semaphores | S1, S2, both sem_init(..., 1) |
| Actors | T_L (100) holds S1 and S2; T_H (140) waits on S1; T_X (160) waits on S2 |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | T_L : sem_wait(S1), sem_wait(S2) | holdsem(T_L) = {S2:1, S1:1} |
| 2 | T_H : sem_wait(S1) blocks | prio(T_L) = 140 |
| 3 | T_X : sem_wait(S2) blocks | prio(T_L) = 160 |
| 4 | T_L : sem_post(S1) | S1 count goes to T_H; T_L still holds S2 whose waiter is T_X at 160, therefore prio(T_L) stays 160 |
| 5 | T_L : sem_post(S2) | S2 count goes to T_X; T_L holds nothing; prio(T_L) = 100 |

Pass criteria: step 4 must not drop T_L to 100 (that would re-open the inversion on S2), and step 5 must drop it all the way to 100. Defect signature: restoring straight to base_priority without the holdsem walk.

### PI-05 - Classic priority inversion is bounded

| Field | Value |
| --- | --- |
| Objective | End to end proof that PI bounds the blocking time of T_H by T_L's critical section, not by T_M's CPU burn. |
| Config | CFG-1; also run under CFG-4 to record the un-inherited baseline |
| Actors | T_L (100), T_M (120, pure CPU loop of duration D = 2 s, never touches S), T_H (140) |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | T_L takes S, then enters a critical section of d = 50 ms | holders(S) = {T_L:1} |
| 2 | T_M becomes runnable (CPU loop D) | Under PI: T_M does not run yet, because T_L is about to be boosted |
| 3 | T_H calls sem_wait(S) | prio(T_L) = 140, T_L continues to run ahead of T_M |
| 4 | T_L finishes the critical section and posts | T_H acquires S within d + eps of step 3 |
| 5 | Measure T_H blocking time B | CFG-1: B <= d + eps (eps <= 5 ms). CFG-4 (PI off) reference run: B >= D |

Pass criteria: B measured on CFG-1 must be at least an order of magnitude below the CFG-4 baseline, and [trace] must be L...H...M... on CFG-1 versus L...M...H... on CFG-4.

### PI-06 - Counting semaphore with multiple simultaneous holders

| Field | Value |
| --- | --- |
| Objective | All holders of a counting semaphore are boosted, and each is restored as it posts. |
| Config | CFG-1 (needs SEM_PREALLOCHOLDERS >= 3) |
| Semaphore | S = sem_init(&S, 0, 3) |
| Actors | T_L1, T_L2, T_L3 (100, 105, 110), T_X (160) |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | T_L1..T_L3 each sem_wait(S) | count(S) = 0, holders(S) = {T_L1:1, T_L2:1, T_L3:1} |
| 2 | T_X : sem_wait(S) blocks | count(S) = -1; all three holders boosted to 160 |
| 3 | T_L1 : sem_post(S) | count(S) = 0, T_X acquires and is added as holder; T_L1 restored to 100; T_L2, T_L3 restored to 105 / 110 as well, because sem_findhighestwaiter() now finds no waiter (the leaving T_X is excluded) |
| 4 | T_L2, T_L3, T_X post | count(S) = 3, holders empty |

Pass criteria: step 3 restores all holders in one pass (the two-pass A/B walk covers every holder, not only the poster). A holder left at 160 after step 3 is a FAIL. Defect signature: sem_findhighestwaiter() not honouring exclude, which would keep the boost alive because T_X is still linked on g_waitingforsemaphore while priorities are recomputed.

### PI-07 - The released holder entry must not be boosted by its own leftover waiters

| Field | Value |
| --- | --- |
| Objective | Focused regression for the counts <= 0 skip inside the holdsem walk. |
| Config | CFG-1 |
| Semaphores | S1 (value 1), S2 (value 1) |
| Actors | T_L (100) holds S1 and S2; T_H1 (130) and T_H2 (140) wait on S1; nobody waits on S2 |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | T_L takes S1, S2 | holdsem(T_L) = {S2:1, S1:1} |
| 2 | T_H1 waits S1 | prio(T_L) = 130 |
| 3 | T_H2 waits S1 | prio(T_L) = 140, count(S1) = -2 |
| 4 | T_L : sem_post(S1) | T_H2 gets the count. During restore, T_L's S1 entry has counts == 0 and must be skipped, so T_H1 (still waiting on S1) must not re-boost T_L. prio(T_L) = 100. T_L still holds S2 (no waiters). |
| 5 | T_L : sem_post(S2) | prio(T_L) = 100, holders of S2 empty |

Pass criteria: prio(T_L) == 100 immediately after step 4. Defect signature: T_L stuck at 130 - a priority leak that survives until T_L next posts something, and which in a long-running system inflates the priority of a worker thread indefinitely.

### PI-08 - Awakened waiter must be excluded from the recomputation

| Field | Value |
| --- | --- |
| Objective | Verify the exclude argument threaded from sem_restorebaseprio()/sem_canceled() into sem_findhighestwaiter(). |
| Config | CFG-1 |
| Actors | T_L (100) holds S twice (sem_init(&S, 0, 2), T_L takes both counts); T_X (160) waits |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | T_L : sem_wait(S) twice | count(S) = 0, holders(S) = {T_L:2} |
| 2 | T_X : sem_wait(S) blocks | count(S) = -1, prio(T_L) = 160 |
| 3 | T_L : sem_post(S) | T_X receives the count and is unblocked but may still be linked on the wait list during restore. T_L's entry still has counts == 1, so it is examined; sem_findhighestwaiter(S, T_X) must return NULL and prio(T_L) must fall to 100. |
| 4 | T_L : sem_post(S) (second count) | count(S) = 1 (T_X holds one), holders {T_X:1} |

Pass criteria: prio(T_L) == 100 after step 3. Defect signature: without exclude, T_X is found as "highest waiter" of a semaphore it has just been granted, and T_L stays at 160 - the boost never ends.

### PI-09 - Boost is undone when the wait is interrupted by a signal

| Field | Value |
| --- | --- |
| Objective | sem_waitirq() -> sem_canceled() restores holder priorities on EINTR. |
| Config | CFG-1 |
| Actors | T_L (100) holds S; T_H (140) waits; a third task sends SIGUSR1 to T_H |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | T_L takes S | holders(S) = {T_L:1} |
| 2 | T_H : sem_wait(S) blocks | count(S) = -1, prio(T_L) = 140 |
| 3 | Signal delivered to T_H | sem_waitirq(T_H, EINTR): sem_canceled() runs, count(S) back to 0, T_H->waitsem = NULL, prio(T_L) = 100 |
| 4 | T_H returns from sem_wait() | returns ERROR with errno == EINTR; T_H is not a holder of S |
| 5 | T_L : sem_post(S) | count(S) = 1, holders empty, no double increment |

Pass criteria: count(S) is exactly 1 at step 5 (an extra increment in sem_wait()'s resume path would give 2), prio(T_L) == 100 from step 3 onward. Code reference: sem_wait.c resume path comment - clean-up is already done by sem_waitirq.c and must not be repeated.

### PI-10 - Boost is undone on sem_timedwait() timeout

| Field | Value |
| --- | --- |
| Objective | Same as PI-09 via the ETIMEDOUT path, including sem_tickwait(). |
| Config | CFG-1 |
| Actors | T_L (100) holds S for 500 ms; T_H (140) uses sem_timedwait(S, now + 100 ms) |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | T_L takes S | - |
| 2 | T_H : sem_timedwait(S, +100 ms) | prio(T_L) = 140 |
| 3 | Timeout fires | sem_waitirq(T_H, ETIMEDOUT), count(S) = 0, prio(T_L) = 100 |
| 4 | T_H returns | ERROR, errno == ETIMEDOUT, watchdog freed, T_H not a holder |
| 5 | T_L posts at t = 500 ms | count(S) = 1, no waiter, holders empty |
| 6 | Repeat the scenario with sem_tickwait() | Identical results |

Pass criteria: as PI-09, plus no watchdog leak across 100 iterations of the scenario.

### PI-11 - SEM_PRIO_NONE suppresses inheritance completely

| Field | Value |
| --- | --- |
| Objective | PRIOINHERIT_FLAGS_DISABLE must suppress boosting on wait, contribution on restore, and (without BINMGR_RECOVERY) holder retention. |
| Config | CFG-1 and CFG-3 (to cover both branches of sem_setprotocol()) |
| Actors | T_L (100), T_H (140) |

| # | Action | Expected state (CFG-1, BINMGR_RECOVERY=n) | Expected state (CFG-3, BINMGR_RECOVERY=y) |
| --- | --- | --- | --- |
| 1 | sem_init(&S, 0, 1); sem_setprotocol(&S, SEM_PRIO_NONE) | flag set, existing holders destroyed by sem_destroyholder() | flag set, holders retained for fault recovery |
| 2 | T_L : sem_wait(S) | holders(S) empty (no holder recorded) | holders(S) = {T_L:1} |
| 3 | T_H : sem_wait(S) blocks | prio(T_L) = 100 - no boost | prio(T_L) = 100 - no boost (PRIOINHERIT_FLAGS_DISABLE short-circuits sem_boostpriority()) |
| 4 | T_L : sem_post(S) | sem_unblock_task() takes the else-branch: holder looked up and freed directly | same |
| 5 | Cross-check: T_L also holds a PI-enabled S2 with a 150-priority waiter, then S2 is posted | sem_findhighestwaiter(S) must return NULL so the SEM_PRIO_NONE semaphore's waiters never influence T_L's restored priority: prio(T_L) = 100 | same |
| 6 | sem_setprotocol(&S, SEM_PRIO_PROTECT) | returns ERROR, errno == ENOSYS | same |
| 7 | sem_setprotocol(&S, 99) / on an uninitialised semaphore | returns ERROR, errno == EINVAL | same |

Pass criteria: no boost anywhere in the scenario; step 5 is the regression for sem_findhighestwaiter()'s explicit PRIOINHERIT_FLAGS_DISABLE check, which matters only when holders are retained (CFG-3) - the CFG-1 run documents the other branch.

### PI-12 - Signalling semaphore never creates a permanent holder

| Field | Value |
| --- | --- |
| Objective | sem_init(&S, 0, 0) sets FLAGS_SIGSEM, so sem_addholder_tcb() returns early and the classic "awakened waiter becomes a permanent holder" inversion cannot happen. |
| Config | CFG-1, CFG-3 |
| Actors | T_A (100) waits, T_B (120) posts, T_X (160) later waits |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | sem_init(&S, 0, 0) | flags & FLAGS_SIGSEM set; S not added to g_sem_list even on CFG-3 |
| 2 | T_A : sem_wait(S) blocks | count(S) = -1; sem_boostpriority() runs but finds no holder, so nothing is boosted |
| 3 | T_B : sem_post(S) | sem_releaseholder() returns immediately; T_A unblocked; holders(S) remains empty - T_A is not recorded as a holder |
| 4 | T_X : sem_wait(S) blocks | prio(T_A) = 100 - T_A is not boosted, because it is not a holder |
| 5 | T_B : sem_post(S) | T_X unblocked, count(S) = 0, holders still empty |

Pass criteria: holders(S) is empty at every observation point, prio(T_A) never changes. This is the behaviour sem_setprotocol(SEM_PRIO_NONE) documents as the manual fix; the FLAGS_SIGSEM auto-detection must make it unnecessary for zero-initialised semaphores.

### PI-13 - Restore driven from interrupt context

| Field | Value |
| --- | --- |
| Objective | Exercise sem_restorebaseprio_irq() - the single-pass variant with no running-task special case. |
| Config | CFG-1 |
| Actors | T_L (100) holds S; T_H (140) waits; a watchdog callback (ISR context) posts S |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | T_L : sem_wait(S) | holders(S) = {T_L:1} |
| 2 | T_H : sem_wait(S) blocks | prio(T_L) = 140 |
| 3 | Watchdog callback : sem_post(S) from ISR | up_interrupt_context() is true -> sem_restorebaseprio_irq(); T_H gets the count and is added as holder; prio(T_L) = 100 |
| 4 | T_L posts its (now stale) count | Sequence arithmetic: the ISR injected a count, so count(S) ends at 1 with holders(S) = {T_H:1} until T_H posts |
| 5 | T_H posts | count(S) = 2 if the ISR's injected count was never consumed - the scenario asserts the exact expected value computed from the sequence and flags any drift |

Pass criteria: prio(T_L) = 100 right after step 3; no sched_lock() imbalance (sem_unblock_task()'s sched_lock() is a no-op in ISR context, and the corresponding sched_unlock() must not corrupt lockcount); sched_lockcount() of the interrupted task is unchanged across the ISR.

### PI-14 - Counting semaphore posted by a non-holder, single holder present

| Field | Value |
| --- | --- |
| Objective | POSIX allows any task to post a counting semaphore. With exactly one recorded holder, sem_releaseholder() must attribute the release to that holder and return its TCB so the zero-count entry is freed. |
| Config | CFG-1 |
| Actors | T_L (100) is the only holder of S (sem_init(&S, 0, 1)); T_P (120) posts without ever waiting; T_H (140) waits |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | T_L : sem_wait(S) | holders(S) = {T_L:1} |
| 2 | T_H : sem_wait(S) blocks | prio(T_L) = 140, count(S) = -1 |
| 3 | T_P : sem_post(S) | sem_releaseholder() finds T_P is not a holder but total == 1, so T_L's count is decremented and T_L's TCB is returned as htcb; T_H receives the count; prio(T_L) restored to 100; T_L's zero-count entry is freed by sem_restorebaseprio() |
| 4 | T_H : sem_post(S) | count(S) per arithmetic, holders(S) empty |

Pass criteria: after step 3, holders(S) contains only T_H; T_L's stale entry must not survive. Defect signature: attributing the release to T_P leaves T_L recorded as a holder of a semaphore it no longer holds - the entry then boosts T_L on every later waiter and is "released" a second time if T_L is deleted (double increment of semcount).

### PI-15 - Counting semaphore posted by a non-holder, multiple holders present

| Field | Value |
| --- | --- |
| Objective | With more than one holder the released holder is ambiguous; the implementation must keep the existing records rather than guess. |
| Config | CFG-1 |
| Actors | T_L1 (100), T_L2 (105) both hold S (sem_init(&S, 0, 2)); T_P (120) posts; T_H (140) waits |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | T_L1, T_L2 each sem_wait(S) | count(S) = 0, holders(S) = {T_L1:1, T_L2:1} |
| 2 | T_H : sem_wait(S) blocks | count(S) = -1, both holders boosted to 140 |
| 3 | T_P : sem_post(S) | total == 2 -> no holder count is decremented; htcb returned is T_P; T_H gets the count; holders(S) still contains both T_L1:1 and T_L2:1 (plus T_H:1) |
| 4 | Both T_L1 and T_L2 post | Each releases its own count normally; both restored to base priority; holders drained |

Pass criteria: no holder entry is dropped or corrupted in step 3; both low tasks are restored by step 4; sem_nfreeholders() returns to baseline (G-5).

### PI-16 - Explicit reprioritisation discards inheritance history

| Field | Value |
| --- | --- |
| Objective | sched_reprioritize() sets base_priority and clears boost_priority while a boost is active. |
| Config | CFG-1 |
| Actors | T_L (100) holds S; T_H (140) waits; a controller task calls sched_setparam(T_L, 110) while boosted |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | T_L takes S | base(T_L) = 100 |
| 2 | T_H waits | prio(T_L) = 140, base(T_L) = 100 |
| 3 | sched_setparam(T_L, 110) | prio(T_L) = 110, base(T_L) = 110, boost_priority = 0 (inheritance history discarded) |
| 4 | T_L : sem_post(S) | T_H gets the count; prio(T_L) = 110 (its new base), not 100 |
| 5 | Variant: sched_setparam(T_L, 150) at step 3 | prio(T_L) = 150 and it stays 150 after the post |

Pass criteria: after step 4, prio(T_L) == base(T_L) == 110. A drop to 100 means the restore path used a stale base priority.

### PI-17 - Work-queue boost interacts correctly with semaphore restore

| Field | Value |
| --- | --- |
| Objective | sem_restoreholderprio() computes max(boost_priority, base_priority); a boost owned by the kernel work queue (kwork_inherit.c) must not be undone by a semaphore post. |
| Config | CFG-1 with kernel work queue enabled |
| Actors | Worker thread T_W (base 100) boosted to 130 by lpwork_boostworker(); T_W also holds S; T_H (140) waits on S |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | T_W holds S, boost_priority(T_W) = 130, prio(T_W) = 130 | - |
| 2 | T_H waits on S | prio(T_W) = 140 |
| 3 | T_W : sem_post(S) | hpriority = max(130, 100) = 130; no remaining waiter contributes -> prio(T_W) = 130, not 100 |
| 4 | lpwork_restorepriority() | boost_priority = 0, prio(T_W) = 100 |

Pass criteria: step 3 leaves T_W at 130; step 4 completes the restore. Dropping to 100 at step 3 would silently cancel the work-queue's priority boost and re-open a separate inversion.

### PI-18 - Holder container exhaustion degrades safely

| Field | Value |
| --- | --- |
| Objective | With CONFIG_SEM_PREALLOCHOLDERS too small, sem_allocholder() logs "Insufficient pre-allocated holders" and returns NULL; the kernel must keep running. |
| Config | CFG-5 (SEM_PREALLOCHOLDERS = 2) |
| Actors | Four tasks take counts on S = sem_init(&S, 0, 4) |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | Tasks 1 and 2 take counts | Both recorded as holders; sem_nfreeholders() == 0 |
| 2 | Tasks 3 and 4 take counts | sem_wait() still succeeds (count(S) decremented correctly); holders not recorded; sdbg() message emitted |
| 3 | T_X (160) waits | Only the recorded holders are boosted; PI is degraded but no crash |
| 4 | All four post | sem_post() succeeds for the unrecorded holders too (sem_releaseholder() finds nothing to decrement); count(S) arithmetic is exact |
| 5 | Repeat 50 times | sem_nfreeholders() returns to 2 after each round (G-5) |

Pass criteria: G-1, G-3, G-5; the degradation is limited to a missing boost, never to a wrong semcount or a freed-twice container.

### PI-19 - Built-in single holder (SEM_PREALLOCHOLDERS = 0)

| Field | Value |
| --- | --- |
| Objective | The sem->holder built-in container path must behave identically for the single-holder (mutex-like) case. |
| Config | CFG-2 |
| Actors | T_L (100), T_H (140), plus a second low task T_L2 for the overflow probe |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | Run PI-01 unchanged on CFG-2 | Identical results |
| 2 | Run PI-04 unchanged on CFG-2 | Identical results (holdsem list still links the two built-in containers of S1 and S2) |
| 3 | T_L and T_L2 both take counts of sem_init(&S, 0, 2) | Only T_L occupies sem->holder; T_L2 is not recorded; behaves as PI-18 step 2 |
| 4 | Both post | count(S) exact, sem->holder.htcb == NULL at the end |

Pass criteria: no divergence from CFG-1 for the single-holder scenarios; the sem_foreachholder() built-in path must skip the container when htcb == NULL.

### PI-20 - Nested holding: transitive inversion is not propagated (design limit)

| Field | Value |
| --- | --- |
| Objective | Document and lock down the current, non-transitive behaviour so that any future change is a deliberate one. |
| Config | CFG-1 |
| Semaphores | A, B (both value 1) |
| Actors | T_L (100) holds A; T_M (120) holds B and waits on A; T_H (140) waits on B |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | T_L takes A | - |
| 2 | T_M takes B, then waits on A | prio(T_L) = 120 (boosted by T_M) |
| 3 | T_H waits on B | prio(T_M) = 140. prio(T_L) stays 120 - the boost of T_M does not re-run sem_boostpriority() on A, so the chain is not walked |
| 4 | T_L posts A | T_M acquires A; prio(T_L) = 100 |
| 5 | T_M posts A then B | T_H acquires B; prio(T_M) = 120 |

Pass criteria: the exact priorities above, including prio(T_L) == 120 (not 140) at step 3. The test is a characterisation test: if a future change adds transitive propagation, this scenario must be updated together with that change, never silently.

### PI-21 - pthread mutex protocol attribute

| Field | Value |
| --- | --- |
| Objective | pthread_mutexattr_setprotocol() must map onto sem_setprotocol() for the underlying semaphore. |
| Config | CFG-1 |
| Actors | T_L (100), T_H (140) |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | Mutex M1 initialised with PTHREAD_PRIO_INHERIT (default) | M1.sem.flags has FLAGS_SEM_MUTEX, no PRIOINHERIT_FLAGS_DISABLE |
| 2 | T_L locks M1, T_H blocks on it | prio(T_L) = 140; count(M1.sem) = -1 (still < 2, G-6) |
| 3 | T_L unlocks | prio(T_L) = 100, T_H owns M1, count(M1.sem) = 0 |
| 4 | Mutex M2 initialised with PTHREAD_PRIO_NONE | PRIOINHERIT_FLAGS_DISABLE set |
| 5 | Repeat steps 2-3 on M2 | prio(T_L) never changes |
| 6 | Recursive type (PTHREAD_MUTEX_RECURSIVE), T_L locks twice | nlocks == 2, count(M2.sem) still < 2 (G-6), single holder entry with counts == 1 |

Pass criteria: G-6 must hold at every step - a mutex semaphore must never be observed with semcount >= 2.

### PI-22 - Priority restored when a *waiter* is deleted

| Field | Value |
| --- | --- |
| Objective | Deleting the boosting waiter must undo the boost, via sem_recover() -> sem_canceled(). |
| Config | CFG-1 |
| Actors | T_L (100) holds S; T_H (140) waits; a controller deletes T_H |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | T_L takes S | - |
| 2 | T_H waits on S | prio(T_L) = 140, count(S) = -1 |
| 3 | task_delete(T_H) | sem_recover() sees TSTATE_WAIT_SEM: sem_canceled(T_H, S) restores prio(T_L) = 100; count(S) = 0; T_H->waitsem = NULL; task_state deliberately left as TSTATE_WAIT_SEM |
| 4 | T_L : sem_post(S) | count(S) = 1, no waiter found, holders empty |
| 5 | Variant: two waiters T_H (140) and T_X (160); delete T_X | prio(T_L) falls from 160 to 140, not to 100 - T_H still waits and must still boost |

Pass criteria: step 3 gives exactly prio(T_L) == 100, step 5 gives exactly 140. Step 5 is the key discrimination: a restore that ignores the remaining waiters would re-open the inversion for T_H.

## 6. Part B - Semaphore Recovery Scenarios

### 6.1 Group B1 - Mutex usage recovery

### SR-01 - Delete a task that is *waiting* on a mutex semaphore

| Field | Value |
| --- | --- |
| Objective | sem_recover()'s waiting-task branch, including the mutex semcount < 2 invariant. |
| Config | CFG-1 |
| Semaphore | M, a pthread_mutex_t (FLAGS_SEM_MUTEX, initial semcount = 1) |
| Actors | T_L (100) owns M; T_H (140) blocks on it; a controller deletes T_H |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | T_L locks M | count(M) = 0, holders(M) = {T_L:1} |
| 2 | T_H locks M and blocks | count(M) = -1, prio(T_L) = 140 |
| 3 | pthread_cancel(T_H) / task_delete(T_H) | sem_recover(): sem_canceled() -> prio(T_L) = 100; count(M) = 0; DEBUGASSERT(count(M) < 2) holds; T_H->waitsem = NULL; T_H state stays TSTATE_WAIT_SEM so it is still correctly unlinked from g_waitingforsemaphore by the delete path |
| 4 | sem_release_all(T_H) (same call) | T_H held nothing -> no-op, count(M) unchanged at 0 |
| 5 | T_L unlocks M | count(M) = 1, holders empty, no waiter woken |

Pass criteria: G-6 at every step; count(M) is exactly 1 at the end (a spurious extra increment would make it 2 and trip the mutex assert). Defect signature: incrementing semcount in both sem_recover() and the delete path's unlink logic.

### SR-02 - Delete the *holder* of a mutex semaphore, one waiter present

| Field | Value |
| --- | --- |
| Objective | sem_release_all() must release the count and hand it to the waiting task, exactly as sem_post() would. |
| Config | CFG-1 |
| Actors | T_L (100) owns M; T_H (140) blocks; controller deletes T_L |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | T_L locks M | count(M) = 0, holders(M) = {T_L:1} |
| 2 | T_H blocks on M | count(M) = -1, prio(T_L) = 140 |
| 3 | task_delete(T_L) | sem_recover(): T_L is not waiting -> straight to sem_release_all(). sem_freeholder() unlinks the entry from both M and T_L->holdsem; count(M) becomes 0; because count(M) <= 0, the first matching waiter T_H is made a holder (sem_addholder_tcb()), waitsem = NULL, up_unblock_task(T_H) |
| 4 | T_H resumes | sem_wait() returns OK (errno is neither EINTR nor ETIMEDOUT); holders(M) = {T_H:1} |
| 5 | T_H unlocks M | count(M) = 1, holders empty |

Pass criteria: T_H must actually resume (G-7). A sem_release_all() that only bumps semcount without waking the waiter leaves T_H blocked forever with a positive-looking count - the primary defect this scenario targets.

### SR-03 - pthread_cancel() of a mutex owner marks the mutex inconsistent

| Field | Value |
| --- | --- |
| Objective | Robust-mutex path: pthread_mutex_inconsistent() on thread exit. |
| Config | CFG-1, PTHREAD_MUTEX_ROBUST (default when PTHREAD_MUTEX_DEFAULT_UNSAFE=n) |
| Actors | TH_A owns M; TH_B blocks on M; TH_C locks M later |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | TH_A : pthread_mutex_lock(M) | M on TH_A->mhead, mutex->pid = TH_A |
| 2 | TH_B : pthread_mutex_lock(M) blocks | count(M.sem) = -1 |
| 3 | pthread_cancel(TH_A) | pthread_mutex_inconsistent(TH_A): M removed from mhead, _PTHREAD_MFLAGS_INCONSISTENT set, pthread_sem_give(&M.sem) -> TH_B unblocked |
| 4 | TH_B returns from lock | returns EOWNERDEAD, and TH_B does own the mutex |
| 5 | TH_C : pthread_mutex_lock(M) while still inconsistent | returns EOWNERDEAD immediately (checked before taking the semaphore) |
| 6 | TH_B : pthread_mutex_consistent(M) then unlock | flag cleared, subsequent locks succeed normally |

Pass criteria: EOWNERDEAD at steps 4 and 5; count(M.sem) never reaches 2 (G-6); after step 6 the mutex is fully usable. Variant: with PTHREAD_MUTEX_STALLED configured, step 4 must not report EOWNERDEAD, and the scenario asserts the stalled semantics instead.

### SR-04 - Mutex owner *exits normally* without unlocking

| Field | Value |
| --- | --- |
| Objective | pthread_exit() follows the same recovery path as cancellation; the semaphore count must not be released twice (once by pthread_mutex_inconsistent() and once by sem_release_all()). |
| Config | CFG-1 |
| Actors | TH_A owns M and returns from its start routine; TH_B blocks |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | TH_A locks M, TH_B blocks | count(M.sem) = -1 |
| 2 | TH_A returns / pthread_exit() | pthread_mutex_inconsistent() posts M.sem -> TH_B becomes holder, count(M.sem) = 0; M is removed from TH_A->mhead so it is no longer associated with the dying thread |
| 3 | task_recover(TH_A) -> sem_release_all(TH_A) | TH_A->holdsem no longer contains M (the holder entry was freed when pthread_sem_give() completed the post) -> no second increment |
| 4 | Final check | count(M.sem) = 0 with TH_B holding; count(M.sem) < 2 (G-6) |
| 5 | TH_B unlocks | count(M.sem) = 1, holders empty |

Pass criteria: exactly one release. A double release shows up as count(M.sem) == 1 at step 4 while TH_B still believes it owns the mutex - and as an assert on the next lock/unlock pair.

### SR-05 - Delete a task holding several mutexes and semaphores

| Field | Value |
| --- | --- |
| Objective | sem_release_all() drains the entire holdsem list, in any order, waking one waiter per semaphore. |
| Config | CFG-1 |
| Semaphores | M (mutex), S1 (counting, value 1), S2 (counting, value 1) |
| Actors | T_L (100) holds all three; T_H1 (130) waits on M; T_H2 (135) waits on S1; nobody waits on S2 |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | T_L acquires M, S1, S2 | holdsem(T_L) has 3 entries; prio(T_L) = 100 |
| 2 | T_H1 waits on M; T_H2 waits on S1 | prio(T_L) = 135 |
| 3 | task_delete(T_L) | sem_release_all() walks holdsem until empty: M -> count = 0, T_H1 woken and made holder; S1 -> count = 0, T_H2 woken and made holder; S2 -> count = 1, no waiter |
| 4 | T_H1, T_H2 resume and release | count(M) = 1, count(S1) = 1, count(S2) = 1, all holder lists empty |
| 5 | sem_nfreeholders() | Back to the pre-scenario value (G-5) |

Pass criteria: both waiters resume; holdsem(T_L) is empty (the while loop terminates - a sem_freeholder() that fails to unlink from holdsem would spin forever, which this scenario detects as a hang / watchdog reset).

### SR-06 - Delete a task holding multiple counts of one semaphore

| Field | Value |
| --- | --- |
| Objective | Verify count accounting when a single holder entry carries counts > 1. |
| Config | CFG-1 |
| Semaphore | S = sem_init(&S, 0, 3) |
| Actors | T_L (100) takes 2 counts; T_H (140) waits |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | T_L : sem_wait(S) twice | count(S) = 1, holders(S) = {T_L:2} |
| 2 | T_H : sem_wait(S) three times (last one blocks) | T_H takes the remaining count, then blocks: count(S) = -1, holders(S) = {T_L:2, T_H:1} |
| 3 | task_delete(T_L) | sem_release_all() frees T_L's single holder entry and increments semcount once (the loop is per holder entry, not per count): count(S) = 0, and the waiter T_H is granted the count |
| 4 | Observation | count(S) reflects one returned count although T_L had taken two. Record the observed value; the expected value for a leak-free implementation is one further count returned. |
| 5 | Restart the scenario 20 times | Each round leaks one count in the same way, so count(S) after a full drain is monotonically decreasing - the scenario reports the delta explicitly rather than only pass/fail |

Pass criteria: the scenario must be run and its measured delta recorded. G-1, G-5 and G-7 must hold unconditionally. The count delta of step 4 is reported as a defect if it is non-zero; it is the intended detection target of this scenario, and it must not be silently accepted by the harness.

### SR-07 - Delete the holder while other holders remain boosted

| Field | Value |
| --- | --- |
| Objective | sem_release_all() does not run the priority-restore pass; verify what happens to the *other* boosted holders of the same counting semaphore. |
| Config | CFG-1 |
| Semaphore | S = sem_init(&S, 0, 2) |
| Actors | T_L1 (100) and T_L2 (105) each hold one count; T_X (160) waits; controller deletes T_L1 |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | T_L1, T_L2 take one count each | count(S) = 0, both recorded as holders |
| 2 | T_X waits | count(S) = -1, prio(T_L1) = prio(T_L2) = 160 |
| 3 | task_delete(T_L1) | sem_release_all() frees T_L1's entry, count(S) = 0, T_X granted the count and unblocked |
| 4 | Observation on T_L2 | T_L2 is still recorded as a holder and its boost is not recomputed by the recovery path. Record prio(T_L2); it is expected to remain 160 until T_L2 itself posts. |
| 5 | T_L2 : sem_post(S) | Normal restore path runs: prio(T_L2) = 105 |

Pass criteria: G-1..G-7 must hold at step 5. Step 4 is an explicit measurement point: a lingering boost between steps 3 and 5 is reported with the measured duration, because a long-lived T_L2 keeps running at 160 in the meantime. The scenario fails outright if prio(T_L2) is still 160 after step 5.

### 6.2 Group B2 - Signalling usage and holder accounting

### SR-08 - Delete a task waiting on a signalling semaphore

| Field | Value |
| --- | --- |
| Objective | Recovery for FLAGS_SIGSEM semaphores, where there are no holder records at all. |
| Config | CFG-1, CFG-3 |
| Semaphore | S = sem_init(&S, 0, 0) -> FLAGS_SIGSEM |
| Actors | T_A (120) waits; T_B (100) is the intended signaller; controller deletes T_A |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | T_A : sem_wait(S) blocks | count(S) = -1, holders(S) empty |
| 2 | task_delete(T_A) | sem_recover(): TSTATE_WAIT_SEM branch; sem_canceled() walks an empty holder list (no-op); count(S) = 0; waitsem = NULL; sem_release_all() is a no-op |
| 3 | T_B : sem_post(S) | count(S) = 1; no waiter; no holder recorded for the signalling semaphore |
| 4 | A new task T_C : sem_wait(S) | Succeeds immediately, count(S) = 0, still no holder |

Pass criteria: count(S) follows the arithmetic exactly; holders(S) is empty throughout; no assert from sem_canceled()'s DEBUGASSERT(sem->semcount <= 0) (step 2 enters with count(S) == -1, which satisfies it).

### SR-09 - Delete the *signaller* of a signalling semaphore

| Field | Value |
| --- | --- |
| Objective | Because no holder is recorded for a FLAGS_SIGSEM semaphore, deleting the task that was supposed to post it cannot be recovered by sem_release_all(); the waiter must be released by the application's own timeout, not by the kernel. |
| Config | CFG-1 |
| Actors | T_B (100) will post; T_A (120) waits with sem_timedwait(S, +200 ms) |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | T_A : sem_timedwait(S, +200 ms) | count(S) = -1 |
| 2 | task_delete(T_B) before it posts | sem_release_all(T_B) finds nothing - T_B was never a holder of S (FLAGS_SIGSEM); count(S) unchanged at -1 |
| 3 | Timeout at 200 ms | sem_waitirq(T_A, ETIMEDOUT): count(S) = 0, T_A returns ERROR / ETIMEDOUT |
| 4 | Variant with plain sem_wait() | T_A stays blocked indefinitely - documented as the expected consequence of signalling usage, and the reason sem_timedwait()/sem_tickwait() is mandatory for signalling protocols that must survive peer death |

Pass criteria: step 3 recovers cleanly with the exact count; step 4 must be recorded as a blocked-forever observation and must not assert or corrupt the wait list. G-7 is waived for the step-4 variant, which terminates by deleting T_A.

### SR-10 - Signalling semaphore is not registered for binary-manager recovery

| Field | Value |
| --- | --- |
| Objective | sem_init() registers into g_sem_list only when semcount != 0; therefore signalling semaphores are outside the scope of binary_manager_release_binary_sem(). |
| Config | CFG-3 (BINMGR_RECOVERY=y) |
| Semaphores | S_sig = sem_init(&S_sig, 0, 0) (kernel region), S_cnt = sem_init(&S_cnt, 0, 1) (kernel region) |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | Initialise both | S_cnt is present in g_sem_list; S_sig is not |
| 2 | sem_destroy(&S_sig) | sem_unregister() is skipped (the FLAGS_SIGSEM guard) - and correctly so, since it was never registered; g_sem_list stays consistent |
| 3 | sem_destroy(&S_cnt) | sem_unregister() runs; g_sem_list no longer contains it |
| 4 | Walk g_sem_list | List is well formed (no dangling flink), traversal terminates |
| 5 | Repeat init/destroy 100 times | g_sem_list length returns to its baseline every cycle |

Pass criteria: g_sem_list traversal must terminate at every observation point; a stale entry after step 3 would make binary_manager_release_binary_sem() walk freed memory.

### SR-11 - Holder list integrity across sem_destroy() with stranded holders

| Field | Value |
| --- | --- |
| Objective | sem_destroyholder() must unlink each holder entry from both the semaphore's list and the holder task's holdsem list. |
| Config | CFG-1, CFG-2 |
| Actors | T_L1 (100) and T_L2 (105) hold counts of S (sem_init(&S, 0, 2)) and each also holds a second semaphore S2 |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | Both tasks take a count of S and of S2 | holdsem(T_L1) and holdsem(T_L2) each have 2 entries |
| 2 | sem_destroy(&S) while both still hold counts | sdbg("Semaphore destroyed with holders"); sem_foreachholder(sem, sem_recoverholders, NULL) frees both entries; count(S) reset to 1 then flags = 0 |
| 3 | Inspect holdsem(T_L1) / holdsem(T_L2) | Each has exactly 1 entry left (S2) - the S entries were unlinked from the task lists too |
| 4 | T_L1 posts S2 | sem_restoreholderprio() walks holdsem and must not dereference a freed S entry |
| 5 | sem_nfreeholders() | Both containers returned to the free list (G-5) |

Pass criteria: step 4 must not fault. A sem_freeholder() that skipped the holdsem unlink leaves a dangling hpholder->sem pointer that the very next priority restore dereferences.

### SR-12 - Recovery does not disturb TSTATE_WAIT_SEM bookkeeping

| Field | Value |
| --- | --- |
| Objective | sem_recover() deliberately clears waitsem but leaves task_state == TSTATE_WAIT_SEM, because the TCB is still linked in g_waitingforsemaphore and the delete path relies on that to unlink it. |
| Config | CFG-1 |
| Actors | T_H1 (140), T_H2 (135) both waiting on S held by T_L (100); controller deletes T_H1 |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | T_L holds S; T_H1 and T_H2 block | count(S) = -2, wait list order T_H1, T_H2, prio(T_L) = 140 |
| 2 | task_delete(T_H1) | Inside sem_recover(): waitsem = NULL, task_state still TSTATE_WAIT_SEM, count(S) = -1, prio(T_L) = 135 (T_H2 still boosts) |
| 3 | Delete completes | T_H1 removed from g_waitingforsemaphore; the list head is now T_H2 and the list is well formed |
| 4 | T_L : sem_post(S) | sem_unblock_task() scans the wait list; it must find T_H2 (and never a stale T_H1 whose waitsem is NULL); T_H2 acquires, prio(T_L) = 100 |
| 5 | T_H2 posts | count(S) = 1, holders empty |

Pass criteria: step 4 must wake T_H2; the stcb->waitsem != sem scan must skip the recovered TCB safely if the delete has not yet unlinked it.

### SR-13 - sem_reset() with waiters present

| Field | Value |
| --- | --- |
| Objective | Error-path recovery API: sem_reset() hands counts to waiters through sem_post() and then forces the count. |
| Config | CFG-1 |
| Actors | T_H1 (140), T_H2 (135), T_H3 (130) all waiting on S (sem_init(&S, 0, 0), so FLAGS_SIGSEM); controller calls sem_reset(&S, 2) |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | Three waiters block | count(S) = -3 |
| 2 | sem_reset(&S, 2) | sched_lock() + critical section held; loop posts twice: T_H1 then T_H2 are woken in priority order; count(S) = -1 after the loop; since count < 0, the final semcount = count assignment is skipped |
| 3 | After sem_reset() returns | T_H3 is still blocked with count(S) = -1 - the documented behaviour when waiters outnumber the requested count |
| 4 | sem_reset(&S, 5) again | T_H3 woken; count(S) becomes 4 (5 - 1 handed out) |
| 5 | Negative parameters | sem_reset(&S, -1) -> ERROR, errno == EINVAL; sem_reset(NULL, 1) -> ERROR, EINVAL; sem_reset() on a destroyed semaphore -> ERROR, EINVAL |
| 6 | Repeat step 2 on a FLAGS_SEM_MUTEX semaphore with count = 2 | Must be treated as a misuse: G-6 (count < 2) is the binding invariant and the scenario records the assert |

Pass criteria: the exact counts above; no context switch inside the reset loop escapes the sched_lock() (verified by the trace: the reset caller must complete before either woken waiter runs).

### SR-14 - Signal racing with recovery on the same semaphore

| Field | Value |
| --- | --- |
| Objective | A signal delivered to a waiter at the same moment it is being deleted must not double-increment semcount; sem_waitirq() and sem_recover() both guard on task_state == TSTATE_WAIT_SEM. |
| Config | CFG-1 |
| Actors | T_L (100) holds S; T_H (140) waits; controller sends SIGUSR1 and calls task_delete(T_H) back to back, with the order swapped between runs |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | T_L holds, T_H waits | count(S) = -1 |
| 2 | Run A: signal, then delete | sem_waitirq() runs first: count(S) = 0, waitsem = NULL, T_H unblocked (state no longer TSTATE_WAIT_SEM). sem_recover() then takes the *else* path and only calls sem_release_all() -> count(S) stays 0 |
| 3 | Run B: delete, then signal | sem_recover() runs first: count(S) = 0, waitsem = NULL, state left as TSTATE_WAIT_SEM. A subsequent sem_waitirq() on the same TCB would re-enter the branch - the scenario asserts it does not happen after the TCB is unlinked, and that count(S) is still 0 |
| 4 | T_L posts | count(S) = 1 in both runs |
| 5 | Loop each run 500 times with jittered delays | count(S) after each iteration is exactly 1; no assert |

Pass criteria: count(S) == 1 at step 4 in every iteration. A value of 2 means one of the two paths incremented without the state guard - and on a mutex semaphore that is an immediate DEBUGASSERT(semcount < 2) failure.

### SR-15 - Binary-manager fault recovery, waiting threads

| Field | Value |
| --- | --- |
| Objective | binary_manager_recover_tcb() mirrors sem_recover() for every thread of a faulted user binary. |
| Config | CFG-3 |
| Actors | User binary U with threads U1 (waiting on kernel semaphore K, mutex-flagged) and U2 (waiting on user semaphore S_u); kernel task T_K (140) also waits on K |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | U1 holds nothing and waits on K; U2 waits on S_u | count(K) < 0, count(S_u) < 0 |
| 2 | Fault injected in U | binary_manager_deactivate_binary() walks BIN_NRTLIST, calling binary_manager_recover_tcb() per thread |
| 3 | Per waiting thread | ASSERT(sem != NULL && semcount < 0) holds; sem_canceled() restores holder priorities; semcount++; DEBUGASSERT(count(K) < 2) for the mutex-flagged K; waitsem = NULL |
| 4 | Watchdog / mqueue resources | tcb->waitdog deleted and nulled; nwaitnotempty / nwaitnotfull decremented if applicable |
| 5 | BM_DEACTIVATE_TASK() | Each thread removed from its state list; no thread of U remains runnable |

Pass criteria: no assert; T_K's priority is restored if it had boosted anything; counts of K and S_u are exactly one higher than at step 1.

### SR-16 - Binary-manager fault recovery, kernel semaphores held by the binary

| Field | Value |
| --- | --- |
| Objective | binary_manager_release_binary_sem() walks g_sem_list and releases every kernel semaphore held by a thread of the faulted binary, waking waiters. |
| Config | CFG-3 |
| Actors | Thread U1 of binary U holds kernel semaphore K (registered, non-signalling); kernel task T_K (140) waits on K |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | U1 acquires K | holders(K) = {U1:1}, K present in g_sem_list |
| 2 | T_K waits on K | count(K) = -1, prio(U1) boosted to 140 |
| 3 | Binary U faults | After per-thread recovery, binary_manager_release_binary_sem(bin_idx) matches holder->htcb->group->tg_binidx == bin_idx: count(K)++ -> 0; mutex assert count(K) < 2 holds; sem_unblock_task(K, U1) hands the count to T_K and runs sem_restorebaseprio() |
| 4 | T_K resumes | Owns K, holders(K) = {T_K:1}; U1's holder entry freed |
| 5 | Cross-check with a second kernel semaphore K2 held by a *different* binary | K2 untouched: count(K2) and holders(K2) unchanged |
| 6 | Cross-check with a signalling kernel semaphore K_sig | Not present in g_sem_list, therefore not released - recorded as the documented scope limit (see SR-10) |

Pass criteria: T_K resumes (G-7); only the faulted binary's semaphores are touched (step 5); g_sem_list traversal completes.

### SR-17 - Holder container leak check under repeated create/delete

| Field | Value |
| --- | --- |
| Objective | Stress the whole recovery machinery and prove that holder containers, holdsem links and g_sem_list entries are conserved. |
| Config | CFG-1, CFG-3, CFG-5 |
| Actors | A creator loop spawning T_L (100) and T_H (140) pairs |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | Record baseline sem_nfreeholders(), g_sem_list length, free heap | baseline captured |
| 2 | 1000 iterations of: create S and mutex M; T_L acquires both; T_H blocks on one of them (alternating); delete T_L at a random point (holding / waiting / mid-post); let T_H finish; sem_destroy(S), pthread_mutex_destroy(M) | Each iteration ends with all tasks gone and both semaphores destroyed |
| 3 | Compare against baseline | sem_nfreeholders(), g_sem_list length and free heap all back to baseline |
| 4 | Throughout | No assert, no watchdog reset, no unbounded growth in the semaphore-history ring's error counters |

Pass criteria: exact baseline restoration at step 3; any monotonic drift over the 1000 iterations is a FAIL even if no assert fires.

### SR-18 - pthread_join helper semaphores are recovered

| Field | Value |
| --- | --- |
| Objective | pthread_create() sets SEM_PRIO_NONE on pjoin->data_sem and pjoin->exit_sem; recovery of a cancelled joiner or joinee must not leave these stuck. |
| Config | CFG-1 |
| Actors | TH_A (100) is joinable; TH_B (120) calls pthread_join(TH_A) |

| # | Action | Expected state |
| --- | --- | --- |
| 1 | TH_B : pthread_join(TH_A) blocks on exit_sem | count(exit_sem) = -1; no PI boost of TH_A (PRIOINHERIT_FLAGS_DISABLE) |
| 2 | pthread_cancel(TH_B) (the joiner) | sem_recover(TH_B): count(exit_sem) = 0, waitsem = NULL; sem_canceled() finds no holders to restore |
| 3 | TH_A exits | pthread_exit() completes without blocking on the now-unjoined pjoin; join structure freed exactly once |
| 4 | Variant: cancel TH_A (the joinee) while TH_B is joining | TH_B returns from pthread_join(); PTHREAD_CANCELED reported as the exit value |
| 5 | Repeat 200 times | No heap growth, no stuck task (G-7) |

Pass criteria: neither variant leaves a task blocked; the join structure is freed exactly once in both.

## 7. Traceability Matrix

| Source element | Covered by |
| --- | --- |
| sem_wait.c PI block (sem_boostpriority under sched_lock) | PI-01, PI-02, PI-03, PI-11, PI-12 |
| sem_wait.c resume path (EINTR / ETIMEDOUT discrimination) | PI-09, PI-10, SR-14 |
| sem_wait.c / sem_trywait.c mutex assert (semcount < 2) | PI-21, SR-01, SR-03, SR-13 |
| sem_holder.c:sem_allocholder() / sem_findorallocateholder() | PI-06, PI-18, PI-19 |
| sem_holder.c:sem_freeholder() (both lists) | SR-05, SR-11, SR-17 |
| sem_holder.c:sem_foreachholder() (prealloc and built-in paths) | PI-06, PI-19, SR-11 |
| sem_holder.c:sem_boostholderprio() (strict > test) | PI-02, PI-03 |
| sem_holder.c:sem_findhighestwaiter() - exclude | PI-06, PI-08, PI-22 |
| sem_holder.c:sem_findhighestwaiter() - PRIOINHERIT_FLAGS_DISABLE early return | PI-11 |
| sem_holder.c:sem_restoreholderprio() - holdsem walk | PI-04, PI-07, PI-17 |
| sem_holder.c:sem_restoreholderprio() - counts <= 0 skip + DEBUGASSERT | PI-03, PI-07 |
| sem_holder.c:sem_restoreholderprio() - max(boost_priority, base_priority) | PI-16, PI-17 |
| sem_holder.c:sem_restorebaseprio_task() two-pass A/B | PI-01, PI-06 |
| sem_holder.c:sem_restorebaseprio_irq() | PI-13 |
| sem_holder.c:sem_addholder_tcb() - FLAGS_SIGSEM early return | PI-12, SR-08, SR-09 |
| sem_holder.c:sem_addholder_tcb() - SEM_VALUE_MAX guard | PI-18, SR-06 |
| sem_holder.c:sem_releaseholder() - non-holder poster, single holder | PI-14 |
| sem_holder.c:sem_releaseholder() - non-holder poster, multiple holders | PI-15 |
| sem_holder.c:sem_release_all() - waiter wake-up | SR-02, SR-05, SR-07 |
| sem_holder.c:sem_release_all() - count accounting per holder entry | SR-06 |
| sem_holder.c:sem_canceled() | PI-09, PI-10, PI-22, SR-01, SR-08, SR-15 |
| sem_holder.c:sem_destroyholder() | SR-11 |
| sem_post.c:sem_unblock_task() - PI-disabled else-branch | PI-11 |
| sem_post.c:sem_post() - SEM_VALUE_MAX assert | PI-13, SR-13 |
| sem_recover.c - waiting branch | SR-01, SR-08, SR-12, SR-14 |
| sem_recover.c - sem_release_all() call | SR-02, SR-05, SR-06, SR-07 |
| sem_waitirq.c | PI-09, PI-10, SR-09, SR-14 |
| sem_setprotocol.c - all three protocols and error paths | PI-11, PI-21 |
| sem_destroy.c - FLAGS_SIGSEM unregister guard | SR-10 |
| sem_reset.c | SR-13 |
| sem_list.c (sem_register / sem_unregister) | SR-10, SR-16, SR-17 |
| lib/libc/semaphore/sem_init.c - FLAGS_SIGSEM auto-set, registration rule | PI-12, SR-10 |
| pthread_mutexinit.c - FLAGS_SEM_MUTEX + protocol | PI-21 |
| pthread_mutex.c / pthread_mutexinconsistent.c / pthread_mutexconsistent.c | SR-03, SR-04 |
| pthread_create.c - SEM_PRIO_NONE on join semaphores | SR-18 |
| task_recover.c | SR-01, SR-02, SR-05 |
| sched_reprioritize.c | PI-16 |
| binary_manager_recovery.c:binary_manager_recover_tcb() | SR-15 |
| binary_manager_load.c:binary_manager_release_binary_sem() | SR-16 |

## 8. Execution Plan and Exit Criteria

### 8.1 Execution order

1.  Smoke - PI-01, PI-11, PI-12, SR-01, SR-02 on CFG-1. A failure here blocks the rest.

2.  Functional - all PI-xx and SR-xx on CFG-1.

3.  Configuration sweep - PI-01, PI-04, PI-06, PI-19, SR-05, SR-11 on CFG-2; PI-11, SR-10, SR-15, SR-16, SR-17 on CFG-3; PI-05 on CFG-4 (baseline); PI-18, SR-17 on CFG-5.

4.  Stress - SR-14 (500 iterations), SR-17 (1000 iterations), SR-18 (200 iterations), plus PI-05 repeated 100 times with random T_M load.

### 8.2 Exit criteria

| Id | Criterion |
| --- | --- |
| E-1 | Every scenario in sections 5 and 6 executed on at least CFG-1, with the configuration sweep of 8.1 completed |
| E-2 | No PASS recorded for any scenario that violated a global rule G-1..G-7 |
| E-3 | Every measurement scenario (SR-06, SR-07, SR-09 step 4, PI-20) has its measured value recorded in the run report, not merely a pass/fail flag |
| E-4 | Stress runs complete with baseline-restored holder counts, g_sem_list length and free heap |
| E-5 | Every entry of the traceability matrix in section 7 is exercised by at least one executed scenario |

### 8.3 Reporting

Each run produces, per scenario: scenario id, configuration id, iterations, measured priorities / counts at every observation point, the execution trace string, and PASS / FAIL / MEASURED. Scenarios PI-20, SR-06, SR-07 and SR-09 are characterisation scenarios: their measured values must be diffed against the previous run, and any change must be tied to a deliberate kernel change.